February 28, 2012

Craig Dukes

Ralf Ehrlich

Stephen Goadhouse

Bridget Mason

Documentation for the 
NOA Power Distribution System 

Version 3.3

Figure 1The Power Distribution Box (PDB)

# Content

1	Content	3

2	Tables	5

3	Figures	6

4	Power Distribution System	7

4.1	Changes in Version 3.3	7

4.2	Overview	8

4.3	Power supplies	10

5	Detailed Descriptions	12

5.1	Power Distribution Box	13

5.2	External Cables	15

5.3	Backplane	17

5.4	Indicator Card	18

5.5	DCM Card	19

5.6	FEB Card	20

5.7	High Voltage Patch Panel	21

5.8	Low Voltage Breaker Box	24

5.9	Proposal for the Far Detector Grounding System	26

6	Components	29

6.1	Power Distribution Box	30

6.2	External Cables	42

6.3	Backplane	47

6.4	Indicator Card	48

6.5	DCM Card	49

6.6	FEB Card	51

6.7	High Voltage Patch Panel	52

6.8	Low Voltage Breakout Box	57

7	Pictures	59

7.1	Power Distribution Box	60

7.2	Backplane	65

7.3	Indicator Card	67

7.4	DCM Card	68

7.5	FEB Card	71

7.6	High Voltage Patch Panel	72

7.7	Low Voltage Breakout Box	76

8	Figures and Schematics	78

8.1	Power Distribution Box	79

8.2	Backplane	80

8.3	Indicator Card	82

8.4	DCM Card	83

8.5	FEB Card	86

8.6	High Voltage Patch panel	91

9	Datasheets, Dimensional Drawings, etc.	93

9.1	Power Supplies	94

9.2	Power Distribution Box Enclosure	105

9.3	High Voltage Patch panel Enclosure	112

9.4	Connectors	115

9.5	Switches	166

9.6	Fuses, Resistors, etc.	174

9.7	Cables	183

9.8	Dimensional Drawings for Busbars, Backplane and Cards	203

9.9	Faceplates for Cards	210

# Tables

Table 1 NOA electronics channel counts	8

Table 2 Power requirements for the detector electronics.	9

Table 3 Power Distribution Box current design capacity	14

Table 4 Color Scheme of the External Cables	15

Table 5 Requirements for the Bus Bars of the Power Distribution System	17

Table 6 Mapping between  the pins of the REDEL connectors in the HV patch panel.	23

Table 7 Components of Power Distribution Box	30

Table 8 External Cables	42

Table 9 Components of Backplane	47

Table 10 Components of Indicator Card	48

Table 11 Components of DCM Card	49

Table 12 Components of FEB Card	51

Table 13 Components of the High Voltage Patch Panel	54

# Figures

Figure 1The Power Distribution Box (PDB)	2

Figure 2 Layout of the Power Distribution System.	8

Figure 3 Power budget of the Power Distribution System.	11

Figure 4 The pinout for the PDB remote sense connector	16

Figure 5 Header Pinout for the FEB connector	19

Figure 6 Front Panel Layout of DCM Card	19

Figure 7 Header Pinout for the DCM connector	19

Figure 8 Front Panel Layout of FEB Card	20

Figure 9 Header Pinout for the FEB connector	20

Figure 10 Positions of the pins at the REDEL connector.	23

Figure 11NEC Table of Gounding Conductor Specifications	26

Figure 12 Block Diagram of the Proposed Ground Layout	27

Figure 13 Power Distribution Box Low Voltage Connectors and Backplane with Bus Bars	60

Figure 14 Power Distribution Box v. 3.3	60

Figure 15 Power Distribution Box (Low Voltages Breakers and Backplane Connectors)	61

Figure 16 Power Distribution Box (Ground Connectos, and Backplane with Bus Bars)	62

Figure 17 Power Distribution Box (Low Voltage Connectors and Back Panel with High Voltage Connector and Sense Cable Connector)	63

Figure 18 Power Distribution Box (Backpanel)	64

Figure 19 Backplane	66

Figure 20 Backplane (first 5 slots from the left side)	66

Figure 21 Indicator Card	67

Figure 22 Indicator Card	67

Figure 23 DCM Card	69

Figure 24 DCM Card Front Panel	69

Figure 26 FEB Card	72

Figure 27 High Voltage Power Supply and High Voltage Patch panel.	73

Figure 28 Picture of HV patch panel with top cover removed	74

75

Figure 29 A closer view of the Patch Panel PCB. The silk screen indicates the HV channel and the pinout to the REDEL connector.	75

Figure 30 An unpopulated Patch Panel PCB	75

Figure 31 The rear panel of the High Voltage Patch Panel, with channel and PDB position labels.	75

Figure 32 Chassis mounted in a relay rack	76

Figure 33 PL506 power supply and chassis	76

Figure 34 Chassis with power supply installed in relay rack.	76

Figure 35 Underside of chassis mounted in relay rack.	76

Figure 36 Inside of the breaker box.	77

Figure 34 Back of the breaker box	77

Figure 39 Circuit Diagram for Backplane	80

Figure 40 Circuit Diagram for Indicator Card	82

Figure 41 Circuit Diagram for DCM Card	83

Figure 42 Circuit Diagram for FEB Card	86

Figure 43 Circuit diagram for HV patch panel.	91

Figure 44 Schematic of the Patch Panel PCB	92

# Power Distribution System

## Changes in Version 3.3

There have been several notable changes in this version, which describes the Power Distribution System to be implemented in the Far Detector site. These are:

- A proposal for the Ground Layout for the Far Detector has been added (see page 26). The proposed layout was designed after conferring with Marvin Johnson, an expert in electrical grounding and noise reduction. The proposed layout uses both copper grounding strips along the trenches on the top and side of the detector and 6 AWG copper wires. The number of connection points has been increased from the NDOS layout to reduce the pick-up of long wavelength noise.

- The Low Voltage Breaker Box has been redesigned to increase safety and improve ease of installation and maintenance (see page 24). The source lines for all low voltage channels are protected by marine breakers rated at 100A (for 3.3V) and 60A (for 24V). These are connected to marine connectors for ease of installation. Swivel nuts are no longer used, and have been replaced by angled ring terminals.

- The Power Distribution Box (PDB) bus bars have been redesigned to include tabs for connecting the optional ground braid. These braids can be disconnected to provide a floating ground to the PDB. The bus bars have also been resized. Previously, the 24V bus bars were had the same current capacity as the 3.3V bus bars. This was unnecessary since the maximum current of the 24V channels is much less than that of the 3.3V channels. See page 203 for dimensional drawings.

- The DCM card schematic and layout has been updated to include additional over-voltage protection circuitry (see page 83).

- The High Voltage Patch Panel has been redesigned (see page 21). The new design uses a printed circuit board to provide an isolated connection to ground for each HV channel to avoid ground loops (see page 91 for schematic). The enclosure has been changed to a standard 1U 19” rack mount enclosure. REDEL connectors are now used on all HV connection points from the HV power supply to the HV patch panel.

- Swivel nuts, previously used in the PDB to connect the source cables to the marine breakers, have been replaced with ring terminals. This was done to improve the assembly process and to reduce potential connectivity issues.

- The back panel of the PDB has been redesigned to allow access to the sense fuses from outside of the enclosure (see page 109). This will improve ease of maintenance.

## Overview

The NOA power distribution system (PDS) supplies power to four different electronic components used in the readout: front-end boards (FEBs), avalanche photodiodes (APDs), thermoelectric coolers (TECs), and data concentrator modules (DCMs). The power distribution system consists of: the power supplies and their racks, the power distribution boxes that fan out the power to the FEBs, APDs, TECs, and DCMs, and the power cables and their cable trays. Two similar systems are needed: one for the Near Detector and one for the Far Detector. The number of each component is given in Table 1. A schematic is shown in Figure 2. The voltage, current, and power requirements for the five components are given in Table 2. Turn-on currents have not been measured.

| Module | Near Detector | Far Detector |
| --- | --- | --- |
| FEB | 497 | 11,160 |
| TEC | 497 | 11,160 |
| APD | 497 | 11,160 |
| DCM | 10 | 180 |
| TDU | 1 | 15 |

Table 1
NOA electronics channel counts

Figure 2
Layout of the Power Distribution System.

Each power distribution box feeds 3.3V (FEB), 24V (TEC), and 450V (APD) via a single cable to a maximum of 66 front-end boards, and 24V via a single cable to the nearby data concentrator module. The APD voltage to the power distribution boxes is provided by Wiener MPOD cards situated in a Wiener mainframe. The FEB, TE cooler, and DCM power are provided by Wiener PL506 power supplies. The channel counts are for the NOA Far Detector only.

In the design of the power distribution system we have adhered to the electrical design standards outlined in Electrical Design Standards for Electronics to be used in Experiment Apparatus at Fermilab and the Fermilab ES&H Manual 5050: Electrical Safety.

|  |  |  | Power Distribution Box |  |  |
| --- | --- | --- | --- | --- | --- |
| Module | Voltage | Current | Channels | Total current | Power |
| FEB | 3.3 V | 1.0 A | 60 - 64 | 60 - 64 A | 211 W |
| TEC | 24 V | 0.30 A | 60 - 64 | 18 - 20 A | 480 W |
| APD | 350 - 450 V | 40 μA | 60 - 64 | 2.4 - 2.6 mA | 2 W |
| DCM | 24 V | 1.25 A | 1 | 1.25 A | 30 W |

Table 2
Power requirements for the detector electronics.

All power to the front-end electronics is distributed by power distribution boxes (PDBs – see ) which are situated on the detector. A single 6-conductor, 18AWG cable from each PDB output carries the high voltage needed by the APDs, the 24V needed by the TE coolers, as well as the 3.3V needed by the FEBs, and their return currents. All the electronic components and the power supplies float: ground reference is at the PDBs. A maximum of 66 FEBs and 1 DCM are served by each PDB.

## Power supplies

The 450V to the PDBs is provided by two Wiener MPOD HV-EX high voltage power supplies System mainframes using 15 (6 installed in each mainframe and 3 spares) ISEG EHS F6-05x 156-F (see Figure 27 and page 101) floating 16-channel cards. Each of the 16 channels of the ISEG is individually programmable from 0-500V, and provides a maximum of 15mA of current. A multi-conductor cable takes the 16 HV channels from each ISEG card to a nearby patch panel (see Figure 27), and shielded tri-axial RG59 cables with BNC connectors go from the patch panel to the PDBs. Each Wiener HV channel feeds a single PDB.

The low voltages (3.3V and 24V) needed by the FEBs, DCMs, and TE coolers, are provided by the Wiener PL506 Power Supply System (see page 95). Each Wiener PL506 chassis has six floating individually programmable power supply modules: three 2-7V channels, each rated to 115A and 550W, and three 12-30V channels, each rated to 23A and 550W. Hence each PL506 crate feeds power to three adjacent PDBs: the 3.3V via 2AWG cables and the 24V via 6AWG cables. The cables can safely carry the maximum rated current of the Wiener supplies without the need for fuses; nevertheless they will be protected with 60A (for the 6AWG cables) and 100A (for the 2AWG cables) breakers at a breaker panel close to the power supply. The Wiener low voltage supplies have a remote sensing feature which allows the voltages at the PDBs to be set to their desired values. This feature is needed because the different cable lengths from the Wieners to the PDBs produce non-negligible differences in the voltage drops. The required power, including cable losses is shown in Figure 3. The longest cable runs are about 16 m, corresponding to one-way voltage drops of 0.51V and 0.56 V, respectively for the 3.3V and 24V lines. Voltage drops on the 18AWG cables running from the PDBs to the FEBs are much smaller, resulting in a voltage range at the FEBs between 3.41-3.30V, well within the allowed range of the FEB voltage regulators, which protect the FEBs from inadvertent application of the highest voltage (7V) the Wiener supplies can provide. A total of 122 kW of power is consumed by the FEBs, DCMs, and TE coolers (the APDs consume no appreciable power); 19 kW is lost in the cables and 3 kW is lost in the PDBs to heat.

Both of the Wiener power supplies are remotely controllable with Ethernet interfaces, and have programmable output voltages and current limits.

Figure 3 Power budget of the Power Distribution System.

The power budget of a type AB diblock is shown, including the power supply losses, cable losses, and losses in the Power Distribution Boxes. The required set voltages to produce a nominal 3.3V and 24V at the FEBs is also given.

# Detailed Descriptions

## Power Distribution Box

A total of 180 far detector and 10 near detector power distribution boxes (PDBs) are needed (not including spares; there are 18 spares for the far detector). To fit within the tight space constraints of both detectors the power distribution boxes employ a standard DIN 3U subrack with a custom backplane that feeds 3.3V, 24V, and 450V to 18 cards: 16 FEB cards that feed 3.3V, 24V, and 450V power to the front-end boards (FEBs), 1 DCM card that feeds 24V power to a data concentrator module, and 1 Indicator card that has LED lamps that indicate of the on/off status. A schematic of the PDB can be found in Figure 38.

The low-voltage power (24V and 3.3V) is input through connectors [O2][O3] (see Figure 13 and Figure 18) on the side of the PDB. The “source cables” [C1][C2] go to front-panel circuit breakers rated at 60A [O6] and 100A [O7] respectively for the 24V and 3.3V lines (see Figure 15), which can also be used to locally power off the PDB, although the operating procedure will be to always power off the crate at the supply before handling it or its cards. The breakers are connected to the source busbars [O19a][O19b] (which are soldered to the backplane) via the cables [C3][C4] (see Figure 16). The “return cables” [C5][C6] connect the input connectors [O2][O3] with the ground busbars [O20a][O20c] (see Figure 13). Both ground busbars are connected to the return busbar [O20b] via ground braids [C8][C9], and cable [C7] connects the return busbar with the ground connector bar [O18] (see Figure 16), which is in electrical contact with the chassis of the PDB, and has a stud to connect the ground the PDB to the building ground.

The high-voltage power is supplied to the PDB via a triaxial bulkhead [O1] at the backpanel [E3] of the PDB (see Figure 17 and Figure 18). The high voltage source cable runs from this triaxial bulkhead to the front panel switch [O5a], and from there to the backplane connector [O28]. Since the triaxial bulkhead allows only a maximum wire size of 22AWG and the front panel switch requires a minimum size of 18AWG, two sections [C10a] (22AWG) and [C10b] (18AWG) of the source cable are required which are connected with each other by a crimp-on wire connector [O11]. A third section [C10c] of the source cable goes from the switch to the backplane connector. The return cable [C11] forms a twisted pair with the source cables, and goes from the triaxial bulkhead [O1] to the backplane connector [O28].

The DCM source cable [C12] start at the “input” stud of the 60A breakers (see Figure 15), i.e. it is directly connected to the low voltage source cables [C1] and switching off the breaker does not lead to an interruption. The DCM return cables [C13] start at the 24V return busbars [O20a]. The DCM cables are connected to the backplane connector [O27].

The sense cables for the 24V and 3.3V source ([C14][C16]) start at the “input” studs of the low voltage breakers (see Figure 15) – just like the DCM source cable, i.e. they are directly connected to the low voltage source cables [C1][C2] and switching off the breakers does not lead to an interruption. The sense cables for the 24V and 3.3V return ([C15][C17]) start at the return busbars. All 4 sense cables go the sense cable connector [O4a] at the backpanel of the PDB (see Figure 17 and Figure 18). The sense cables are fused with inline fuses (fuseholder [O8] and fuse [O9]) which are connected to the cables via crimp-on wire connectors [O10] (see Figure 17).

The box has been designed to provide the currents given in Table 3, all of which are roughly a factor of two greater than the expected currents. The individual cards, described below, are fused at 1/0.75 = 1.33 the maximum rating. The trace widths are set to handle currents 1.4 times the fused value.

| Item | 24V | 3.3V | 450 V |
| --- | --- | --- | --- |
| Backplane | 70A | 132A | N/A |
| FEB card 1 channel | 1.0A | 2.0A | N/A |
| FEB card 4 channels | 4.0A | 8.0A | N/A |
| DCM card output to DCM | 6.0A | N/A | N/A |

Table 3
Power Distribution Box current design capacity

## External Cables

The external NOA cables are listed in Table 7, and the color scheme is listed in Table 4. There are three power cables to run from the power supplies in the catwalk to the PDBs alongside the detector: 3.3V, 24V, and 450V. One sense cable runs from the PDBs to the power supplies and is used for remote sensing of the voltage at the PDBs. The 3.3V, 24V, and 450V power from the PBBs to the FEBs is carried by a single six-conductor cable, as is the 24V power from the PDBs to the DCMs.

| TEC (24V) | source | return |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | yellow | black |  |  |  |  |
| FEB (3.3V) | source | return |  |  |  |  |
|  | red | black |  |  |  |  |
| APD (450V) | source | return | ground |  |  |  |
|  | inner conductor | intermediate shield | outer shield |  |  |  |
|  | jacket: yellow |  |  |  |  |  |
| Sense | FEB (3.3V) |  | TEC (24V) |  |  |  |
|  | source | return | source | return |  |  |
|  | green | white | red | black |  |  |
|  | jacket: gray |  |  |  |  |  |
| PDB - DCM | source | return |  |  |  |  |
|  | red | black |  |  |  |  |
|  | jacket: white |  |  |  |  |  |
| PDB - FEB | FEB (3.3V) |  | TEC (24V) |  | APD (450V) |  |
|  | source | return | source | return | source | return |
|  | orange | yellow | blue | brown | red | black |
|  | jacket: black |  |  |  |  |  |

Table 4
Color Scheme of the External Cables

For each PDB, the power cables supplying 24V [P1a][P2a] and 3.3V [P3a][P4a] to the PDBs go from the Wiener low voltage power supplies to the breaker panels [P11] with 60A [P9] and 100A [P10] breakers. The power supplied and the breaker panels are both installed in relay racks. The power cables [P1b][P2b][P3b][P4b] connect the breaker panel with the Power Distribution Boxes, where the source and return cables of the same voltage form twisted pairs. The power cables are terminated with the low voltage connectors [P5][P6] which mate with the connectors [O2][O3] at the back of PDB (see Figure 18).

The multi-conductor power cables [H2] from the Wiener MPOD high voltage power supplies will be routed to a patch panel on the same relay racks. The cables running from the patch panel to the PDBs are triaxialial cables [P14]. They are terminated by triaxial connectors [P15] which are used to connect them to the triaxial bulkhead [O1] at the back of the PDB.

Sense cables run from the low voltage power supplies to the Power Distribution Boxes. The cables consist of two individually shielded, twisted pairs [P16]. The sense lines are terminated on the supply end in the manufacturer’s pluggable terminal strips. At the Power Distribution Box they are terminated in a D-sub style plug [P17a], which are attached to the receptacle [O4a] at the back of the PDB (see Figure 18). The pinout of the connector is shown in Figure 4. The sense lines are fused at 0.25A [O9] inside of the PDB (see Figure 17) to prevent damage to the lines.

Figure 4 The pinout for the PDB remote sense connector

Pin 4	24V source	red conductor

Pin 9	24V return		black conductor

Pin 2	3.3V source	green conductor

Pin 6	3.3V return	white conductor

The power cables going from the PDBs to the FEBs are 6-conductor, 18AWG, non-paired, tray cable, with a PVC jacket [P18]. They are terminated on both sides with a 6 position plug [P19]. At the PDB side, they are plugged into the FEB connectors at the FEB cards (see Figure 9). They carry the 3.3V, 24V, and 450V to the FEBs where the other end of the cables is plugged in.

The cable [P20] is a 2-conductor 18AWG, which carries the 24V power to the DCM. It is terminated with a 2 position plug on both ends. At the PDB side, it is plugged into the DCM connector of the DCM card (see Figure 7), the other end is plugged into the DCM.

## Backplane

The backplane feeds the power to the all of the cards. The 3.3V, 24V, and ground feeds are all done using copper bus bars that are soldered to the pc board. The 3.3V RET and 24V RET lines are connected to the ground bus bar, although if it were decided to float the PDB, those connections can easily be severed. To reduce the possibility of a short, the RET bus bars are on the outside. The bus bar cross sectional area has been set to limit the current to be below the 1000A/in2 specification given in Fermilab’s Electrical Design Standards (see Table 5). Note that this capacity exceeds the maximum output capacity of the power supplies. A schematics of the backplane can be found in Figure 39, and dimensional drawings of the busbars are at pages 204 and 205 .

| Item | Design Current | Required Area | Minimum Size | Actual Size |
| --- | --- | --- | --- | --- |
| 3.3V | 132A | 0.132in2 | 1/8" × 1.056" | 1/8" × 1.150" |
| 3.3V RET | 132A | 0.132in2 | 1/8" × 1.056" | 1/8" × 1.650" |
| 24V | 70A | 0.070in2 | 1/8" × 0.560" | 1/8" × 1.150" |
| 24V RET | 70A | 0.070in2 | 1/8" × 0.560" | 1/8" × 1.650" |
| Ground | 202A | 0.202in2 | 1/8" × 1.616" | 1/8" × 1.650" |

Table 5
Requirements for the Bus Bars of the Power Distribution System

## Indicator Card

The Indicator Card gets inserted into the card slot next to the LV breakers and HV switch. It has LEDs that show which voltages are on: 3.3V (green – FEB), 24V (yellow – TEC, and 450V (red – APD). A schematics of the Indicator Card can be found in Figure 40.

## DCM Card

A special card, called the DCM card, feeds 24V power to the associated DCM, and has two spare FEB power outlets (see also page 20). The power to the DCM is tapped off before the 24V breaker, allowing the DCM to be powered on while the 24V to the TE coolers is off. An LED lamp indicates that the power is on. Just like for the FEB card, each 3.3V and 24V output is individually fused and has LED indicators.

In the original design a switch allowed the power to be turned off. A small enough switch could not be found that handles the increased current rating, and we decided to abandon the switch altogether.

A schematics of the DCM Card can be found in Figure 41.

Figure 5
Header Pinout for the FEB connector

Figure 6
Front Panel Layout of DCM Card

Figure 7
Header Pinout for the DCM connector

## FEB Card

Each FEB card provides power to 4 FEBs, allowing a maximum of 64 FEBs to be powered through one PDB by the 16 FEB cards. (Two more FEBs can be powered extra channels in another card called the DCM card.) Each 3.3V and 24V output is individually fused and has LED indicators. There is no LED indicator for the 450V outputs. Transient voltage suppressors on the 3.3V lines protect the FEBs from over voltage due to an out of regulation power supply. A current limiter for the HV lines was added to the design in Version 3 of the FEB card. This was added as a safety feature under the recommendation of Steve Chappa, and it limits the current to below 1 mA. It has the added feature that a short in one front-end board, rather than tripping the Wiener, and putting all of the HV channels served by the PDB down, only kills the channel in question. However, the limiter does produced a current-dependent drop in the output voltage. Because of this, and because we wanted channel-by-channel voltage control for individual APDs, a shunt regulator voltage control circuit was added to the front-end board design. As mentioned above, two current limiters were designed and tested; the results can be found in NOA doc-db. A fault analysis of the current limiter circuit was done by John Oliver, the NOA Project Electrical Engineer,  and was approved by Steve Chappa in October 2009. A schematics of the FEB Card can be found in Figure 42.

Figure 8
Front Panel Layout of FEB Card

Figure 9
Header Pinout for the FEB connector

## High Voltage Patch Panel

The HV patch panel is used to distribute the high voltage from the high voltage power supply to the power distribution boxes. The power supply is connected to the patch panel via a 51 conductor cable, supplied by the manufacturer of the power supply. The patch panel supplies the high voltage to each PDB via triaxialial cables. Each power supply has card 16 channels, as does each patch panel. Schematics of the High Voltage Patch panel can be found in Figure 43.

The power supply which is used, is the ISEG EHS F6-05x 156-F (see page 101), which has a maximum voltage of 500 V and a maximum current of 15 mA per channel. It provides 16 channels with floating ground. The power supply has a 51 pin REDEL connector (see Figure 10, Figure 27), with 32 pins used for the source and return of the 16 HV channels (see Table 6). Two additional pins are used for the safety loop. According to the power supplies manual states that, “if the safety loop is active then output voltage on all channels is only present if the safety loop is closed and an external current in a range of 5 to 20 mA of any polarity is flowing into the loop. If the safety loop is opened during operation then the output voltages are shut off without ramp.” The remaining 17 pins are unused.

The connection between the power supply and the patch panel is also made with a 51 pin REDEL connector.

The wires for each pin of the receptacle exit the receptacle at the inside of the patch panel. The wire connects the pins 33 and 34 to form a closed (safety) loop. The wires at pins 1 through 32 are used for the 16 HV channels, where the source wire and return wire of each channel form a twisted pair (see Table 6, Figure 28, and Figure 43).

The other end of the twisted pair wires get connected to triaxial bulkheads which are mounted at the back of the patch panel, where the source wire is soldered into the center conductor, and the return wire is soldered to the intermediate shield of the bulkhead. The triaxial bulkhead is not mounted directly to the chassis of the patch panel, but rather to the printed circuit board that . The insulation sheet is attached inside of the back of the chassis with the screws/nuts/split-lock washers/flat washers. The spacers keep it a distance away from the chassis, which is done to have enough space for the push-in plugs. The triaxial bulk heads are pushed from the outside of the patch panel through holes at the back of the chassis, and through (keyed) holes in the insulation sheet. A nut at the other side of the insulation sheet keeps the triaxial bulkhead in place. In order to avoid electrical contact between the bulkead and the chassis, the holes in the back of the chasses have a sufficiently bigger diameter than the diameter of the bulkheads and triaxial connectors.

The triaxial bulkhead mates with the triaxial connector. These triaxial connectors terminate the triaxial cables at both ends, where the triaxial cables are used to connect the output of the patch panel with the PDB. The outside of the triaxial bulkhead/connector is connected to the outer shield of the triaxial cable, which is connected to the chassis of the PDB and therefore to the ground of the PDB. Since one wants to keep all these “grounds” separate from each other to reduce noise, there is no direct electrical contact between the triaxial bulkheads/connectors and the chassis (see previous paragraph). However, there is a safety concern: It could happen that the outer shield gets disconnected from the PDB (e.g. either by removing the triaxial connector or by accidentally cutting the wire), which means that the outer shield of the triaxial cable is not grounded anymore. If (in this case) the intermediate shield or the center conductor gets in contact with the outer shield, the triaxial bulkhead, triaxial connector and all uninsulated parts of the cable where the outer shield is unprotected can be at an much higher electrical potential than the ground. In order to avoid such a dangerous situation, one has to make sure that the outer shield can never exceed a certain (safe) maximum voltage. Theoretically, this could be done by having a direct connection between the outer shield and the ground of the chassis. However, this may create noise problems, since this would connect the grounds of all PDBs together. A compromise is a connection to the ground via a resistor of 1 k. This reduces the noise problem, but also ensures that the voltage of the outer shield cannot exceed a safe value of 15 V (which comes from the fact that the power supply’s maximum current is 15 mA). Therefore the outer shield of the triaxial bulkhead is connected to a 1 k resistor installed on the PCB. The ground of the PCB is fastened to the chassis via a nut/external-tooth lock washer/flat washer and goes outside of the patch panel. It is then connected to the relay rack ground.

|  | Channel | Pin at power supply | Power supply to patch panel cable |  | Pin at patch panel box |
| --- | --- | --- | --- | --- | --- |
|  | 0 HV | 2 | 21 - | 2 1- | 2 |
|  | 0 RET | 13 | 1 7 - | 1 7 - | 13 |
|  | 1 HV | 3 | 22 - | 22 - | 3 |
|  | 1 RET | 14 | 1 8 - | 1 8 - | 14 |
|  | 2 HV | 4 | 23 - | 23 - | 4 |
|  | 2 RET | 15 | 7 - | 7 - | 15 |
|  | 3 HV | 5 | 24 - | 24 - | 5 |
|  | 3 RET | 16 | 1 9 - | 1 9 - | 16 |
|  | 4 HV | 6 | 25 - | 25 - | 6 |
|  | 4 RET | 17 | 2 - | 2 - | 17 |
|  | 5 HV | 7 | 26 - | 26 - | 7 |
| Figure 10 Positions of the pins at the REDEL connector. | 5 RET | 18 | 8 - | 8- | 18 |
|  | 6 HV | 8 | 27 - | 27 - | 8 |
|  | 6 RET | 19 | 9 - | 9 - | 19 |
|  | 7 HV | 9 | 28 - | 28 - | 9 |
|  | 7 RET | 20 | 3 - | 3 - | 20 |
| unused pins: pin 1 - conductor 20 pin 11 - conductor 10 pins 41,51 - conductor 37 pins 10,12,21, 23-29,31,40,42 | 8 HV | 50 | 29 - | 29 - | 50 |
|  | 8 RET | 39 | 1 1 - | 1 1 - | 39 |
|  | 9 HV | 49 | 3 0 - | 3 0 - | 49 |
|  | 9 RET | 38 | 12 - | 12 - | 38 |
|  | 10 HV | 48 | 3 1 - | 3 1 - | 48 |
|  | 10 RET | 37 | 4 - | 4 - | 37 |
|  | 11 HV | 47 | 3 2 - | 3 2 - | 47 |
|  | 11 RET | 36 | 5 - | 5 - | 36 |
|  | 12 HV | 46 | 3 3 - | 3 3 - | 46 |
|  | 12 RET | 35 | 13 - | 13 - | 35 |
|  | 13 HV | 45 | 3 4 - | 3 4 - | 45 |
|  | 13 RET | 34 | 14 - | 14 - | 34 |
|  | 14 HV | 44 | 3 5 - | 3 5 - | 44 |
|  | 14 RET | 33 | 15 - | 15 - | 33 |
|  | 15 HV | 43 | 3 6 - | 3 6 - | 43 |
|  | 15 RET | 32 | 16 - | 16 - | 32 |
|  | SL 1 | 22 | 6 - | 6 - | 22 |
|  | SL 2 | 30 | 1 - | 1 - | 30 |

Table 6
Mapping between  the pins of the REDEL connectors in the HV patch panel.

## Low Voltage Breaker Box

The LV power supply units WIENER PL 506 consist of two pieces: The power supply itself, and a chassis in which the power supply rests. This chassis gets mounted in the relay rack at the front mounting rails, and is supported by angle brackets at the bottom (in such a way that the airflow of the power supply is not blocked). After the chassis is installed, the power supply gets pushed in to the chassis, and secured by pushing the front handle down.

The front side of the power supply has an input connector for the 240V power line (which comes from the 240V power interruption panels which are located underneath of each power supply), an Ethernet cable connector which is used to control the power supply via SNMP, an USB connector, a display, and a control buttons to turn on/off the power supply and change some of settings of the power supply (see power supply manual).

# The power supply consists of 6 LV channels, where 3 channels are used for 3.5V and the other 3 channels are used for 24V. The 3.5V channels have a hardware limit of 5.8V. Each channel has a source and return connector, and sense cable connector at the back of the power supply. Pushing the power supply into the chassis connects the terminals at the back of the power supply with the connection points on the inside of the chassis, which itself are connected to terminals at the back of the chassis.

The power supply chassis has been modified with a breaker box at the back. The purpose of this breaker box is to simplify the installation process of the LV power supplies/cables at the Far Detector, and to provide a space for the Marine breakers which protect the cables from accidental over-currents.

The source studs (at the back of the chassis) of each channel are connected to Marine breakers which are mounted at the back of the breaker box. 60A breakers were used for the 24V channels, and 100A breakers were used for the 3.5V channels. The other side of the breakers is connected to the source side of the Marine Connectors which are also located at the back of the breaker box. The return studs (at the back of the chassis) of each channel are directly connected to the return side of the Marine Connectors. The power cables which connect the breaker box with the PDBs are also terminated with Marine connectors. Since the Marine connectors for the 3.5V channels and 24V channels have a different shape, the power cables (which go to the PDBs) cannot be plugged into a channel with the wrong voltage.

A cable gauge of 6AWG is used for all power cables of the 24V channels, and 2AWG cables are used for all power cables of the 3.5V channels.

The sense cable connectors at the back of the chassis are connected with 22AWG cables to D-sub connectors at the back of the breaker box. The sense cables which go to the PDBs are shielded and the shields are connect to the housings of the sense cable connectors on both ends of the sense cables (and therefore also to the PDB chassis). In order to prevent the sense cable shields from being connected with each other via the sense cable connector housings at the breaker box (which may create ground loops), the back of the breaker box is made of insulating material.

|  | position in sense cable connector at the back of the chassis | wire color | pin number at D-sub connector at the breaker panel |
| --- | --- | --- | --- |
| 3.5V source | 1 | green | 2 |
| 3.5V return | 2 | white | 6 |
| 24V source | 3 | red | 4 |
| 24V return | 4 | black | 9 |

specifications for the sense cables inside of the breaker box

The breaker box is covered on the top to prevent people from reaching into the box, and to prevent things from falling into the box.

# Proposal for the Far Detector Grounding System

Since the maximum combined current which a PDB can deliver does not exceed 160A, a safety ground connection with a wire gauge of 6AWG is recommended by the NEC (see figure 11). The cross sectional area of 6AWG wires is 0.021 in2. Therefore copper ground strips which are 2” wide and 0.016” thick (McMaster-Carr 9053K39) can be used as ground strips on top of the detector. These wide ground strips are preferred over regular cables to reduce the inductance which may create an additional noise source. The ground strips are placed into the trenches on top of the detector, with two ground strips per diblock. One of the ground strips is used to connect the cable trays and the PDB chassis to ground via 6AWG cables (see figure 12). The ground strips are connected to the upper catwalk via 6AWG cables. All connections to the copper ground strips are first cleaned with Scotch-Brite before they the cables are attached. The connections are also protected against corrosion by using Penetrox-E at the connection points.

Figure 11NEC Table of Gounding Conductor Specifications

Figure 12 Block Diagram of the Proposed Ground Layout

The ground connection for the PDB chassis on the side of the detector is achieved by connection them to the catwalks with the help of 6AWG cables. The PDBs on the side of the detector are supported by a Unistrut support structure. Since the Unistrut support structure is also connected to the catwalks, the PDB chassis need to be insulated from them to avoid ground loops. This is done by mounting them to the side support via a plastic mating fixture. The cable trays on the side of the detector are directly connected to the Unistrut side support, which provides the safety ground for the cable trays.

The PDB chassis are connected to the “ground busbar” inside of the PDB, which is connected to the returns of the 3.5V, 24V, and HV channels, and also to the return of the DCM channel.

The return of the 3.5V, 24V, and HV channels are used as ground at the respective elements on the front end boards, which are connected to the PDB via the 6 conductor PDB-FEB cable (with wires for the source and return for the 3.5V, 24V, and HV channels).

The return of the DCM channel provides the ground reference for the DCM (through the return wire of the PDB-DCM cable). The DCM chassis on the side of the detector is insulated from the Unistrut side support. The DCM on top of the detector rests on the aluminum PDB/DCM table, so that the DCM chassis on top of the detector is not insulated from the PDB chassis. However, the DCM is chassis is not directly connected to the DCM ground.

# The PDB is connected to the HV patch panel via triaxial cables. The outer shield of the triaxial cable is connected to the outside of the triaxial connectors. Plugging them in at the triaxial bulkhead at the PDB, connects the outer shield of the triaxial cable to the PDB chassis. The other cable end gets plugged into a triaxial bulkhead at the HV patch panel. The triaxial bulkheads at the HV patch panels are not directly connected to the chassis; the connection is done via a 1k resistor. This is done for two reasons: If all 16 triaxial bulkheads were directly connected to the HV patch panel chassis, all 16 associated PDB chassis were linked with each other through the outer shields of the triaxial cables – creating multiple ground connections for the PDBs. This could create ground loops and increases the risk of noise. However, the patch panel end of the outer triaxial shields cannot be completely insulated. If they were insulated, and if the triaxial cable gets disconnected from the PDB (e.g. either by removing the triaxial connector or by accidentally cutting the cable), the outer shield is not grounded anymore. If (in this case) the intermediate shield or the center conductor gets in contact with the outer shield, the triaxial bulkhead, triaxial connector and all uninsulated parts of the cable where the outer shield is unprotected can be at a much higher electrical potential than the ground. In order to avoid such a dangerous situation, one has to make sure that the outer shield can never exceed a certain (safe) maximum voltage. This is achieved by a 1k resistor between the outer shield and the patch panel chassis. Using this resistor, insulates the outer shields enough to avoid the ground loop problem, but it also ensures that the voltage of the outer shield cannot exceed a safe value of 15 V (which comes from the fact that the power supply’s maximum current is 15 mA).

# Components

## Power Distribution Box

### Enclosure

| ID | Description | Quantity | Manufacturer / Item # | Distributor / Item # | Comments |
| --- | --- | --- | --- | --- | --- |
| [E1] | Side Panels | 2 | Schroff 34560-193 | --- | -height: 3U -length: 415mm -modifications: see pages 107 (left panel) and 108 (right panel) -part #1 in picture on page 106 |
| [E2] | Mounting Wings | 2 | Schroff 24564-197 | --- | -for 19” relay rack -height: 3U -parts #2 and #3 in picture on page 106 |
| [E3] | Connector Bar | 4 | Schroff 34560-184 | --- | -width:84HP -type L-SL -parts #6 and #6a in picture on page 106 |
| [E4] | Connector Bar | 2 | Schroff 34560-484 | --- | -width:84HP -type L-ST -part #7 in picture on page 106 |
| [E5] | Threaded Strip | 6 | Schroff 34561-384 | --- | -width 84HP -for parts #6 [E3], #6a [E3] and #7 [E4] in picture on page 106 |
| [E6] | Grub Screws for Threaded Strips and Hole Strips | 6 | Schroff 21101-359 | --- | -size M2.5 -length 9mm -for [E5] and [E7] |
| [E7] | Hole Strip | 2 | Schroff 30845-253 | --- | -width 84HP -for part #6 [E3] in picture on page 106 |
| [E8] | Cover Plate | 2 | Schroff like 24560-081, but non-perforared | --- | -width 84HP -length 355mm -part #9 in picture on page 106 |
| [E9] | Back Panel | 1 | similar to Schroff 20848-097 | --- | -width 84HP -height 3U -modifications: see page 109 -part #10 in picture on page 106 |
| [E10] | EMC Contact Strip | 4 | Schroff 24560-236 | --- | -for contact with front and back panels -for part #6 [E3] and #6a [E3] in picture on page 106 |
| [E11] | EMC Contact Strip | 4 | Schroff 24560-246 | --- | -for contact with cover plates -for part #6 [E3] and #6a [E3] in picture on page 106 |

Table 7
Components of Power Distribution Box

| [E12] | Torx Screws | 12 | Schroff 24560-130 | --- | -size M4 -length 14mm -to connect bars [E3] and [E4] with side panels [E1] -part #11 in picture on page 106 |
| --- | --- | --- | --- | --- | --- |
| [E13] | Torx Screws | 24 | Schroff 21101-884 | --- | -size M4 -length 6mm -to connect cover plates [E8] with side panels [E1] -part #12 in picture on page 106 |
| [E14] | Captive Screws | 41 | Schroff 21101-101 | --- | -size M2.5 -length 12.3mm -to connect back panel [E8] and front panels [E18],[E19],[E20] with connector bars [E3] -part #13 in picture on page 106 |
| [E15] | Sleeves for captive screws | 41 | Schroff 21100-464 | --- | -plastic sleeves for captive screws [E14] -part #14 in picture on page 106 |
| [E16] | Card guides | 36 | Schroff 24560-351 | --- | -plastic -to hold FEB, DCM and Indicator cards -plugged into connector bars [E3] and [E4] |
| [E17] | Front panel for Switches | 1 | Front Panel Express | --- | -material aluminum -finish: alodine (back), anodized (front) -thickness: 2.5mm -dimensional drawings: see page 110 |
| [E18] | Front panel for indicator card | 1 | Front Panel Express | --- | -material aluminum -finish: alodine (back), anodized (front) -thickness: 2.5mm -dimensional drawings: see page 211 -screwed on indicator card |
| [E19] | Front panel for DCM card | 1 | Front Panel Express | --- | -material aluminum -finish: alodine (back), anodized (front) -thickness: 2.5mm -dimensional drawings: see page 212 -screwed on DCM card |

Table 7 (continued)
Components of Power Distribution Box

| [E20] | Front panel for FEB card | 16 | eMachineShop | --- | -material aluminum -finish: alodine -thickness: 2.5mm -dimensional drawings: see page 213 -screwed on FEB card |
| --- | --- | --- | --- | --- | --- |
| [E21] | Standoff Screws | 36 | Keystone 8403 | Digikey 8403K-ND | -used to mount the front panels to the DCM, FEB, and Indicator cards |
| [E22] | Screws | 4 | --- | McMaster-Carr 92005A071 | -size: M2.5 -length: 10mm -used to mount the switch front panel [E17] to the connector bars [E3] |
| [E23] | DCM Key | 1 | UVA Shop | --- | -mounted at bottom connector bar [E3] to prevent any other card except the DCM card from being used in this card slot |
| [E24] | Screws | 2 | --- | McMaster-Carr 92000A108 | -size: M2.5 -length: 14mm -used to mount the DCM key [E23] to the bottom connector bar [E3] |
| [E25] | Strain relief bars | 2 | UVA Shop | --- | -length: 17” -tapped holes for #8-32 screws -mounted to the side panels [E1] |
| [E26] | Screws | 4 | --- | McMaster-Carr 90272A192 | -size: #8-32 -length: 3/8” -used to mount the Strain relief bar [E25] to the side panels [E1] |

Table 7 (continued)
Components of Power Distribution Box

### Other Components

| ID | Description | Quantity | Manufacturer / Item # | Distributor / Item # | Comments |
| --- | --- | --- | --- | --- | --- |
| [O1] | High Voltage Connector Bulkhead | 1 | Pomona 5219 | Allied Electronics 885-6069 | -voltage 500 VRMS -mounted at back panel [E9] -datasheet: see page 116 |
| [O2] | Low Voltage Connector | 1 | APP 6800G3 | C W Distribution | -rated for 120A -color: gray -mounted at side panel [E1] -datasheet: see page 119 |
| [O3] | Low Voltage Connector | 1 | APP 6326G5 | C W Distribution | -rated for 175A -color: blue -mounted at side panel [E1] -datasheet: see page 121 |
| [O4a] | Sense Cable Connector | 1 | Tyco Electronics 205203-3 | Allied Electronics 512-0110 | -receptacle (housing for sockets [O4b]) -9 positions -mounted at back panel [E9] with screwblock kit [O4c] -datasheet: see page 123 |
| [O4b] | Socket Contacts | 4 | Tyco Electronics 66504-9 | Allied Electronics 512-2488 | -for sense cable connector [O4a] -for wire sizes 24-20AWG -datasheet: see page 126 |
| [O4c] | Screwblock Assembly Kit | 1 | Tyco Electronics 5205817-3 | Allied Electronics 845-0400 | -for sense cable connector [O4a] -datasheet: see page 128 |
| [O5]/ [O5a] | High Voltage Switch / Contact Block | 1 of each | Sprecher+Schuh D7M-SM22 / D7-X10 and D7-ALM | McMaster-Carr 9209K551 / 9209K127 | -mounted at front panel [E17] -datasheet: see page 167 |
| [O6] | Low Voltage Breaker | 1 | Blue Sea Systems BS 7547 | D & B Marine | -rated for 60A -mounted at front panel [E17] -datasheet: see pages 172 |
| [O7] | Low Voltage Breaker | 1 | Blue Sea Systems BS 7549 | D & B Marine | -rated for 100A -mounted at front panel [E17] -datasheet: see page 172 |

Table 7 (continued)
Components of Power Distribution Box

| [O8] | Fuse holders | 4 | LittelFuse 03453LS2HXLN | Mouser 576-03453LS2HXLN | -for sense cables -panel mount fuseholder -for fuse size 3AB -terminal: lug -datasheet: see page 175 |
| --- | --- | --- | --- | --- | --- |
| [O9] | Fuses | 4 | LittelFuse 0314.500HXP | Mouser 576-0314.500HXP | -for sense cables -cylinder fuse -fast acting, 0.5A, 250V -size 3AB -datasheet: see page 176 |
| [O11] | Crimp-on Wire Connector | 1 | --- | McMaster-Carr 7242K64 | -rated for 600V -used to connect 22AWG and 18AWG high voltage cables -datasheet: see page 137 |
| [O12a] | Ring Terminals | 2 | Waytek 34000 | Waytek 34000 | -wire size: 6AWG -stud size: #10 -material: tin plated copper -for low voltage cables -datasheet: see page 152 |
| [O12b] | Ring Terminals | 2 | Waytek 34001 | Waytek 34001 | -wire size: 6AWG -stud size: ¼” -material: tin plated copper -for low voltage cables -datasheet: see page 152 |
| [O13] | Ring Terminals | 2 | Waytek 35020 | Waytek 35020 | -wire size: 2AWG -stud size: 1/4" -material: tin plated copper -datasheet: see page 152 |
| [O14] | Ring Terminals | 2 | Tyco 330301 | Mouser 571-330301 | -wire size: 2AWG -stud size: #10 -material: tin plated copper -for low voltage cables -datasheet: see page 153 |

Table 7 (continued)
Components of Power Distribution Box

| [O15] | Ring Terminals | 4 | RNYD1-6@E | Allied Electronics 400-0002 | -wire size: 22-16AWG -stud size: M6 (also 1/4") -for sense cables and DCM cables -datasheet: see page 154 |
| --- | --- | --- | --- | --- | --- |
| [O18] | Ground Connector Bar | 1 | Storm Copper | --- | -tin plated copper -dimensional drawing: see page 111 -mounted on side panel [E1] |
| [O19a]/ [O19b] | Bus Bar | 1 of each | Storm Copper |  | -used for 24V source / 3.3V source -tin plated copper -dimensional drawing: see page 201 -soldered on back plane |
| [O20a]/ [O20b]/ [O20c] | Bus Bar | 1 of each | Storm Copper |  | -used for 24V return / ground / 3.3V return -tin plated copper -dimensional drawing: see page 205 -soldered on back plane |
| [O21a] | Screws | 2 | --- | McMaster-Carr 92941A624 | -size: 3/8”-16 -length: 1” -material: brass -used to mount ground connector bar [E18] to side panel [E1] |
| [O21b] | Nuts | 2 | --- | McMaster-Carr 92676A450 | -size: 3/8”-16 -material: brass -used together with [O21a] |
| [O21c] | External-tooth lock washers | 2 | --- | McMaster-Carr 92164A031 | -size: 3/8” -material: phosphor-bronze -used together with [O21a] |

Table 7 (continued)
Components of Power Distribution Box

| [O21d] | Split lock washers | 2 | --- | McMaster-Carr 93496A031 | -size: 3/8” -material: silicon-bronze -used together with [O21a] |
| --- | --- | --- | --- | --- | --- |
| [O21e] | Flat washers | 2 | --- | McMaster-Carr 92916A405 | -size: 3/8”, OD: 7/8” -material: brass -used together with [O21a] |
| [O22] | Screws | 20 | Brikksen 7985A4PH2.5X6 | --- | -size: M2.5 -length: 6mm -used to mount the backplane to the connector bars [E4] |
| [O23a] | Screws | 4 | --- | McMaster-Carr 93516A539 | -size: 1/4"-20 -length: 5/8” -material: silicon-bronze -used for busbars [O19] and [O20] |
| [O23b] | Nuts | 4 | --- | McMaster-Carr 93439A610 | -size: 1/4"-20 -material: silicon-bronze -used together with [O23a] |
| [O23c] | Split lock washers | 4 | --- | McMaster-Carr 93496A029 | -size: 1/4" -material: silicon-bronze -used together with [O23a] |
| [O23d] | Flat washers | 4 | --- | McMaster-Carr 93496A029 | -size: 1/4", OD: 11/16” -material: silicon-bronze -used together with [O23a] |
| [O24] | Ground Braids | 2 x 5.5” | Erico 557240 | --- | -size: 15-16 -connects the bus bars [O20] (see [C8] and [C9]) -datasheet: see page 200 |
| [O25] | Lugs for Ground Braids | 4 | Erico 557180 | --- | -for ground braid size 15-16 -used to terminate the ends of the ground braids [O24] -datasheet: see page 200 |

Table 7 (continued)
Components of Power Distribution Box

| [O26a] | Screws | 4 | --- | McMaster-Carr 92453A294 | -size: #12-24 -length: 3/4" -material: brass -used to connect the ground braids [O24] to the bus bars [O20] |
| --- | --- | --- | --- | --- | --- |
| [O26b] | Nuts | 4 | --- | McMaster-Carr 92671A013 | -size: #12-24 -material: brass -used together with [O26a] |
| [O26c] | Split lock washers | 4 | --- | McMaster-Carr 93496A418 | -size: #12 -material: silicon-bronze -used together with [O26a] |
| [O26d] | Flat washers | 4 | --- | McMaster-Carr 92916A370 | -size: #12, OD: 9/16” -material: brass -used together with [O26a] |
| [O27] | DCM Plug | 1 | Tyco 172165-1 | Mouser 571-1721651 | -2 position socket housing -voltage rating: 600V -connects the DCM cables to the backplane -datasheet: see pages 132 |
| [O28] | HV Plug | 1 | Tyco 172166-1 | Mouser 571-1721661 | -3 position socket housing -voltage rating: 600V -connects the high voltage cables to the -center position is not used -datasheet: see pages 132 |
| [O29] | Connectors Sockets | 4 | Tyco 1-770988-0 | Mouser 571-1-770988-0 | -for wire size 22-18AWG -voltage: 600V -maximum current: 9.5A -used to terminate the DCM cables and high voltage cables and put into the 2/3 pin housings [O27]/[O28] -datasheet: see page 135 |

Table 7 (continued)
Components of Power Distribution Box

| [O30a] | Screws | 2 | --- | McMaster-Carr 90272A249 | -size: #10-24 -length: 1-1/4” -used to mount low voltage connector [O2] to side panel [E1] |
| --- | --- | --- | --- | --- | --- |
| [O30b] | Nuts | 2 | --- | McMaster-Carr 90480A011 | -size: #10-24 -used together with [O30a] |
| [O30c] | Split lock washers | 2 | --- | McMaster-Carr 91102A740 | -size: #10 -used together with [O30a] |
| [O30d] | Flat washers | 2 | --- | McMaster-Carr 91090A103 | -size: #10 -used together with [O30a] |
| [O31a] | Screws | 2 | --- | McMaster-Carr 90272A546 | -size: 1/4"-20 -length: 1-1/4” -used to mount low voltage connector [O3] to side panel [E1] |
| [O31b] | Nuts | 2 | --- | McMaster-Carr 90480A029 | -size: 1/4"-20 -used together with [O31a] |
| [O31c] | Split lock washers | 2 | --- | McMaster-Carr 91104A029 | -size: 1/4" -used together with [O31a] |
| [O31d] | Flat washers | 2 | --- | McMaster-Carr 91090A105 | -size: 1/4" -used together with [O31a] |

Table 7 (continued)
Components of Power Distribution Box

### Internal Cables

| ID | AWG | Color | Length | From | To | Manufacturer or Distributor / Item # | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [C1] | 6 | yellow | 25cm | -120A low voltage connector [O2] (source pin: bottom) -terminated with contact pins APP 6811G3 which are part of the item [O2] | -60A breaker [O6] (bottom stud) -terminated with swivel nut [O16] | CableCo HV6-85-4 | -24V source -max current: 85A -datasheet: see page 184 |
| [C2] | 2 | red | 25cm | -175A low voltage connector [O3] (source pin: bottom) -terminated with contact pins APP 947 which are part of the item [O3] | -100A breaker [O7] (bottom stud) -terminated with swivel nut [O17] | CableCo HV2-140-2 | -3.3V source -max current: 140A -datasheet: see page 184 |
| [C3] | 6 | yellow | 52cm | -60A breaker [O6] (top stud) -terminated with swivel nut [O16] | -top source busbar [O19a] via screw [O23a] -terminated with ring terminal [O12] | CableCo HV6-85-4 | -24V source -max current: 85A -datasheet: see page 184 |
| [C4] | 2 | red | 52cm | -100A breaker [O7] (top stud) -terminated with swivel nut [O17] | -bottom source busbar [O19b] via screw [O23a] -terminated with ring terminal [O13] | CableCo HV2-140-2 | -3.3V source -max current: 140A -datasheet: see page 184 |
| [C5] | 6 | black | 14cm | -120A low voltage connector [O2] (source pin: top) -terminated with contact pins APP 6811G3 which are part of the item [O2] | -top return busbar [O20a] via screw [O23a] (left hole) -terminated with ring terminal [O12] | CableCo HV6-85-0 | -24V return -max current: 85A -datasheet: see page 184 |
| [C6] | 2 | black | 13cm | -175A low voltage connector [O3] (source pin: top) -terminated with contact pins APP 947 which are part of the item [O3] | -bottom return busbar [O20c] via screw [O23a] (left hole) -terminated with ring terminal [O13] | CableCo HV2-140-0 | -3.3V return -max current: 140A -datasheet: see page 184 |
| [C7] | 2 | black | 9cm | -ground (center) busbar [O20b] via screw [O23a] (right hole) -terminated with ring terminal [O13] | -ground connector bar [O18] via screw [O21a] (inside hole) -terminated with ring terminal [O14] | CableCo HV2-140-0 | -ground -max current: 140A -datasheet: see page 184 |
| [C8] | braids |  | 14cm | top return busbars [O20a] (center hole) | ground (center) busbar [O20b] (center hole) | Erico 557240 | -ground -datasheet: see page 200 |
| [C9] | braids |  | 14cm | bottom return busbars [O20c] (center hole) | ground (center) busbar [O20b] (center hole) | Erico 557240 | -ground -datasheet: see page 200 |

Table 7 (continued)
Components of Power Distribution Box

| [C10a] | 22 | orange | 38cm | triaxial bulkhead [O1] (soldered at center conductor) | connected to [C10b] via crimp-on wire connector [O11] | McMaster-Carr 7587K819 | -high voltage source -voltage: 600VAC -datasheet: see page 185 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [C10b]8 | 18 | orange | 12cm | connected to [C10a] via crimp-on wire connector [O11] | high voltage switch [O5a] | McMaster-Carr 71245K101 | -high voltage source -voltage: 600VAC -datasheet: see page 185 |
| [C10c]8 | 18 | orange | 16cm | high voltage switch [O5a] | -3 pin housing [O28] at backplane (top pin - #1) -terminated with socket [O29] | McMaster-Carr 71245K101 | -high voltage source -voltage: 600VAC -datasheet: see page 185 |
| [C11]8 | 22 | blue | 68cm | triaxial bulkhead [O1] (soldered at intermediate shield) | -3 pin housing [O28] at backplane (bottom pin - #3) -terminated with socket [O29] | McMaster-Carr 7587K281 | -high voltage return -voltage: 600VAC -datasheet: see page 185 |
| [C12] | 18 | red | 14cm | -60A breaker [O6] (bottom stud) -terminated with ring terminal [O15] together with cable [C14a] -uses same stud as cable [C1] | -2 pin housing [O27] at backplane (bottom pin - #2) -terminated with socket [O29] | McMaster-Carr 7587K952 | -DCM cable (24V source) -voltage: 300VAC -datasheet: see page 187 |
| [C13] | 18 | black | 10cm | -top return busbar [O20a] via screw [O23a] (left hole) -terminated with ring terminal [O15] together with cable [C15a] -uses same screw as cable [C5] | -2 pin housing [O27] at backplane (top pin - #1) -terminated with socket [O29] | McMaster-Carr 7587K951 | -DCM cable (24V return) -voltage: 300VAC -datasheet: see page 187 |

Table 7 (continued)
Components of Power Distribution Box

| [C14a] | 22 | red | 46cm | -60A breaker [O6] (bottom stud) -terminated with ring terminal [O15] together with cable [C12] -uses same stud as cable [C1] | -inline fuse holder [O8] -connected via crimp-on wire connector [O10] | McMaster-Carr 7587K932 | -sense cable (24V source) -voltage: 300VAC -datasheet: see page 188 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| [C14b] | 22 | red | 8cm | -inline fuse holder [O8] -connected via crimp-on wire connector [O10] | -sense cable connector [O4a] (pin #4) -terminated with socket [O4b] | McMaster-Carr 7587K932 | -sense cable (24V source) -voltage: 300VAC -datasheet: see page 188 |
| [C15a] | 22 | black | 28cm | -top return busbar [O20a] via screw [O23a] (left hole) -terminated with ring terminal [O15] together with cable [C13] -uses same screw as cable [C5] | -inline fuse holder [O8] -connected via crimp-on wire connector [O10] | McMaster-Carr 7587K931 | -sense cable (24V return) -voltage: 300VAC -datasheet: see page 188 |
| [C15b] | 22 | black | 8cm | -inline fuse holder [O8] -connected via crimp-on wire connector [O10] | -sense cable connector [O4a] (pin #9) -terminated with socket [O4b] | McMaster-Carr 7587K931 | -sense cable (24V return) -voltage: 300VAC -datasheet: see page 188 |
| [C16a] | 22 | green | 46cm | -100A breaker [O7] (bottom stud) -terminated with ring terminal [O15] -uses same stud as cable [C2] | -inline fuse holder [O8] -connected via crimp-on wire connector [O10] | McMaster-Carr 7587K45 | -sense cable (3.3V source) -voltage: 300VAC -datasheet: see page 188 |
| [C16b] | 22 | green | 8cm | -inline fuse holder [O8] -connected via crimp-on wire connector [O10] | -sense cable connector [O4a] (pin #2) -terminated with socket [O4b] | McMaster-Carr 7587K45 | -sense cable (3.3V source) -voltage: 300VAC -datasheet: see page 188 |
| [C17a] | 22 | white | 24cm | -bottom return busbar [O20c] via screw [O23a] (left hole) -terminated with ring terminal [O15] -uses same screw as cable [C6] | -inline fuse holder [O8] -connected via crimp-on wire connector [O10] | McMaster-Carr 7587K934 | -sense cable (3.3V return) -voltage: 300VAC -datasheet: see page 188 |
| [C17b] | 22 | white | 8cm | -inline fuse holder [O8] -connected via crimp-on wire connector [O10] | -sense cable connector [O4a] (pin #6) -terminated with socket [O4b] | McMaster-Carr 7587K934 | -sense cable (3.3V return) -voltage: 300VAC -datasheet: see page 188 |

Table 7 (continued)
Components of Power Distribution Box

## External Cables

| ID | Description | Quantity | Manufacturer / Item # | Distributor / Item # | Comments |
| --- | --- | --- | --- | --- | --- |
| [P1a]/ [P1b] | 6AWG welding cable (yellow) | 1 of each | Tristar | ---- | -for 24V source -from PS to breaker panel [a] and from breaker panel to PDB [b] -terminated at PS with ring terminals [P12] -terminated at breakers [P9] at breaker panel [P11] with ring terminals [P7] -terminated at PDB with contact pins APP 6811G3 which are part of the connector [P5] -datasheet: page 189 |
| [P2a]/ [P2b] | 6AWG welding cable (black) | 1 of each | Tristar | ---- | -for 24V return -from PS to breaker panel [a] and from breaker panel to PDB [b] -terminated at PS with ring terminals [P12] -terminated at breakers [P9] at breaker panel [P11] with ring terminals [P7] -terminated at PDB with contact pins APP 6811G3 which are part of the connector [P5] -datasheet: page 189 |
| [P3a]/ [P3b] | 2AWG welding cable (red) | 1 of each | Tristar | ---- | -for 3.3V source -from PS to breaker panel [a] and from breaker panel to PDB [b] -terminated at PS with ring terminals [P13] -terminated at breakers [P10] at breaker panel [P11] with ring terminals [P8] -terminated at PDB with contact pins APP 947 which are part of the connector [P6] -datasheet: page 189 |
| [P4a]/ [P4b] | 2AWG welding cable (black) | 1 of each | Tristar | ---- | -for 3.3V return -from PS to breaker panel [a] and from breaker panel to PDB [b] -terminated at PS with ring terminals [P13] -terminated at breakers [P10] at breaker panel [P11] with ring terminals [P8] -terminated at PDB with contact pins APP 947 which are part of the connector [P6] -datasheet: page 189 |

Table 8
External Cables

| [P5] | Low Voltage Connector | 1 | APP 6800G3 | C W Distribution | -rated for 120A -color: gray -mates with [O2] at PDB -contact pins APP 6811G3 (which are part of the connector [P5]) terminates cables [P1b] and [P2b] -datasheet: see page 119 |
| --- | --- | --- | --- | --- | --- |
| [P6] | Low Voltage Connector | 1 | APP 6326G5 | C W Distribution | -rated for 175A -color: blue -mates with [O3] at PDB -contact pins APP 947 (which are part of the connector [P6]) terminates cables [P3b] and [P4b] -datasheet: see page 121 |
| [P9] | Low Voltage Breaker | 2 | Blue Sea Systems BS 7547 | D & B Marine | -rated for 60A -mounted at breaker panel [P11] -datasheet: see page 172 |
| [P10] | Low Voltage Breaker | 2 | Blue Sea Systems BS 7549 | D & B Marine | -rated for 100A -mounted at breaker panel [P11] -datasheet: see page 172 |
| [P11] | Breaker Box | 1 | --- | --- | - See page 24 for a detailed description - See page |

Table 8 (continued)
External Cables

| [P14] | Triaxial cable | 1 | Belden 9222 004500 | Mouser 566-9222-500-04 | -connects the HV patch panel with the PDB -operating voltage: 1400 VRMS -terminated at both ends with triaxial connectors [P15] -datasheet: see page 190 |
| --- | --- | --- | --- | --- | --- |
| [P15] | Triaxial connector | 2 | Pomona 5218 | Allied Electronics 885-5218 | -operating voltage: 500 VRMS -used to terminate triaxial cables [P14] -mates with triaxial bulkheads [O1] at the PDB and [H10] at the back of the patch panel -datasheet: see page 117 |
| [P16] | Sense cable | 1 | Belden 8723 | --- | -2 twisted pairs of 22AWG -terminated with pins [P17b] in connector [P17a] (at PDB) -mates with receptacle [O4a] at PDB -connects the PDB with the sense cable connectors at the PS -red conductor: 24V source, pin 4 at connector [P17a] -black conductor: 24V return,. pin 9 at connector [P17a] -green conductor: 3.3V source,. pin 2 at connector [P17a] -white conductor: 3.3V return,. pin 6 at connector [P17a] -datasheet: see page 193 |
| [P17a] | Sense cable connector | 1 | Tyco 205204-4 | Allied Electronics 512-0105 | -plug (housing for pins [P17b]) -9 positions -mates with receptacle [O4a] at PDB -for sense cable [P16] -red conductor: 24V source, pin 4 -black conductor: 24V return,. pin 9 -green conductor: 3.3V source,. pin 2 -white conductor: 3.3V return,. pin 6 -datasheet: see page 123 |

Table 8 (continued)
External Cables

| [P17b] | Pins | 4 | Tyco 66506-9 | Allied Electronics 512-2486 | -for sense cable connector [P17a] -for wire sizes 24-20AWG -terminates single conductors of [P16] -datasheet: see page 126 |
| --- | --- | --- | --- | --- | --- |
| [P17c] | Cable Clamps | 1 | Tyco 5748676-1 | Allied Electronics 374-8768 | -for sense cable connector [P17a] -datasheet: see page 130 |
| [P17d] | Jackscrew | 1 pair | Tyco 747784-8 | Allied Electronics 512-2580 | -for sense cable connector [P17a] -datasheet: see page 130 |
| [P18] | FEB cables | 64 | Belden 27600A | VEC | -connects the PDB with the FEBs -6 conductors of 18AWG -voltage rating: 600V -orange conductor: 3.3V (FEB) source, pin 4 at [P19[ -yellow conductor: 3.3V (FEB) return, pin 1 at [P19[ -blue conductor: 24V (TEC) source, pin 5 at [P19[ -brown conductor: 24V (TEC) return, pin 2 at [P19[ -red conductor: 450V (APD) source, pin 6 at [P19[ -black conductor: 450V (APD) return, pin 3 at [P19[ -terminated with sockets [P22] in connector [P19] -datasheet: see page 196 |
| [P19] | FEB plug | 128 | Tyco 172168-1 | Mouser 571-1721681 | -6 position socket housing -voltage rating: 600V -connects the FEB cables [P18] to the PDB and FEBs -orange conductor: 3.3V (FEB) source, pin 4 -yellow conductor: 3.3V (FEB) return, pin 1 -blue conductor: 24V (TEC) source, pin 5 -brown conductor: 24V (TEC) return, pin 2 -red conductor: 450V (APD) source, pin 6 -black conductor: 450V (APD) return, pin 3 -datasheet: see page 132 |

Table 8 (continued)
External Cables

| [P20] | DCM cable | 1 | Advanced Digital Cable 911802 | VEC | -2 conductors of 18AWG -voltage rating: 300V -used to connect the PDB with the DCM -red conductor: 24V source, pin 2 at [P21] -black conductor: 24V return, pin 1 at [P21] -terminated with sockets [P22] in plug [P21] -datasheet: see page 199 |
| --- | --- | --- | --- | --- | --- |
| [P21] | DCM plug | 128 | Tyco 172168-1 | Mouser 571-1721681 | -2 position socket housing -voltage rating: 600V -connects the DCM cables [P20] to the PDB and DCM -red conductor: 24V source, pin 2 -black conductor: 24V return, pin 1 -datasheet: see page 132 |
| [P22] | Connectors Socket | 768+4 | Tyco 1-770988-0 | Mouser 571-1-770988-0 | -for wire size 22-18AWG -voltage: 600V -maximum current: 9.5A -used to terminate the FEB cables [P18] and put into the 6 socket housings [P19] -used to terminate the DCM cables [P20] and put into the 2 socket housings [P21] -datasheet: see page 135 |

Table 8 (continued)
External Cables

## Backplane

| Item | Qty | Reference | Part | PCB Footprint | Description | Dist Order Code | Distrib. | Primary Manufacturer+ P/N | Secondary Manufacturer+ P/N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 18 | J209,J210, J211,J212, J213,J214, J215,J216, J217,J218, J219,J220, J221,J222, J223,J224, J225,J226 | CON24A | connv24w55p217_165_2x12 | Header, 24 POS, 4.2mm, 600VAC/600VDC, 5.0A, Female, Thru-Hole | 15247243 | Arrow | Molex 0015247243 Datasheet: page 143 |  |
| 2 | 1 | J205 | 1-770872-1 | connv2w55p163_1x2 | Header MINI MATE-N-LOK (2 pin, 600VAC/600VDC, 9.5-3A, vert, gold, thru-hole) | A33330-ND | Digikey | Tyco: 1-770872-1 Datasheet: page 137 |  |
| 3 | 1 | J208 | 1-770873-1 | connv3w55p163_1x3 | Header MINI MATE-N-LOK (3 pin, 600VAC/600VDC, 8.5-3A, vert, gold, thru-hole) | A33332-ND | Digikey | Tyco: 1-770873-1 Datasheet: page 137 |  |
| 4 | 1 | R1 | 0 500V 0.5W | RES1000_42 | RES 0 OHM 1/2W JUMPER | 660-Z25YC | Mouser | KOA Speer: Z25YC |  |
| 5 | 1 | R2 | 20M 3.5kV 0.5W | RES600_42 | RES 20M OHM METAL FILM .50W 3.5kVDC 5% | PPCHHJ20MCT-ND | Digikey | Vishay: VR37000002005JR500 |  |
| 6 | 2 | J201,J203 | Busbar_ source | busbar18w200p800_1x18 | Bus bar for voltage sources (1.150 in) | XXX | XXX | Storm Copper: XXX |  |
| 7 | 3 | J202,J204, J207 | Busbar_ return | busbar18w200p800_1x18 | Bus bar for voltage returns and ground (1.650 in) | XXX | XXX | Storm Copper: XXX |  |

Table 9
Components of Backplane

## Indicator Card

| Item | Qty | Reference | Part | PCB Footprint | Description | Dist Order Code | Distrib. | Primary Manufacturer+ P/N | Secondary Manufacturer+ P/N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | C1 | 0.1u 630V | BOXCAP 590 40 710X360 | CAP METAL POLYPRO .1UF 630V 5% | P12158-ND | Digikey | Panasonic: ECW-F6104JL |  |
| 2 | 2 | FS1,FS2 | 0.1A |  | Fuse (LEDs, 0.1A, 250V TR5) | WK3027BK-ND | Digikey | Littelfuse. Wickmann: 37301000410 |  |
| 3 | 2 | F1,F2 | 56000001319 | TR5 | Fuseholder TR5 machine contacts | WK0001-ND | Digikey | Littelfuse. Wickmann: 56000001319 |  |
| 4 | 1 | J1 | 0015246243 | 15-24-6243 SPECIAL | Header, 24 POS, 4.2mm, 600VAC/600VDC, 5.0A, Male, Rt. Ang, Sel. Gold, Thru-Hole | 15246243 | Arrow | Molex: 0015246243 | Molex: 0015249245 Datasheet: page 147 |
| 5 | 1 | R6 | 4.3k 350V 0.5W | RES_1/2W 600 150 36 AX | RES 4.3K OHM 1/2W 5% CARBON FILM | 4.3KH-ND | Digikey | Yageo: CFR-50JB-4K3 |  |
| 6 | 1 | R7 | 820 | RES_1/4W 500 80 31 AX | RES 820 OHM 1/4W 5% CARBON FILM | 820QBK-ND | Digikey | Yageo: CFR-25JB-820R |  |
| 7 | 1 | R8 | 51k 750V 3W | RES_3W 1000 100 46 AX | RES 51k OHM METAL FILM 3W 750VDC 5% | PPC51KW-3JCT-ND | Digikey | Vishay: PR03000205102JAC00 |  |
| 8 | 1 | R9 | 10M 3.5kV 0.5W | RES_1/2W 600 150 36 AX | RES 10M OHM METAL FILM .50W 3.5kVDC 5% | PPCHJ10MCT-ND | Digikey | Vishay: HVR3700001005JR500 |  |
| 9 | 1 | Q1 | STQ1NK80ZR-AP | TO92 100 60 31 123 | MOSFET, N-CH, 800V, 0.3A, TO-92 | 497-6197-1-ND | Digikey | ST Micro: STQ1NK80ZR-AP |  |
| 10 | 1 | D1 | P6KE56A | DIODE_1W 500 80 46 AX 21 | TVS DIODE, OVP, CLAMPING, 48V | 512-P6KE56A | Mouser | Fairchild: P6KE56A |  |
| 11 | 1 | D2 | HLMP-4719 | LED_SSF-LXH100ID | LED, 5MM, YLW DIFFUSED | 516-1326-ND | Digikey | Avago: HLMP-4719 |  |
| 12 | 1 | D4 | HLMP-4740-A00B2 | LED_SSF-LXH100ID | LED, 5MM, GREEN DIFFUSED, RT ANG | 516-1767-ND | Digikey | Avago: HLMP-4740-A00B2 |  |
| 13 | 1 | SU1 | HLMP-5029 |  | HOUSING RA FOR 5MM HIGH DOME LED | 516-1395-ND | Digikey | Avago: HLMP-5029 |  |
| 14 | 1 | D3 | HLMP-D150-C00B2 | LED_SSF-LXH100ID | LED, 5MM, RED DIFFUSED, RT ANG | 516-1768-ND | Digikey | Avago: HLMP-D150-C00B2 |  |
| 15 | 2 | M1,M2 | 7774 | KEYSTONE-7774 | PCB SCREW TERM MOUNTING BRACKET, 4-40 THREAD | 534-7774 | Mouser | Keystone: 7774 |  |

Table 10
Components of Indicator Card

## DCM Card

| Item | Qty | Reference | Part | PCB Footprint | Description | Dist Order Code | Distrib. | Primary Manufacturer+ P/N | Secondary Manufacturer+ P/N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | C3,C8 | 0.1u 630V | BOXCAP 590 40 710X360 | CAP METAL POLYPRO .1UF 630V 5% | P12158-ND | Digikey | Panasonic: ECW-F6104JL |  |
| 2 | 5 | C1,C4,C6, C9,C11 | 0.1u 50V | DISCCAP 200 80 31 200X100 | Capacitor 3.3V, 24V (0.1uF, 50V) | 478-3193-ND | Digikey | AVX: SR215E104MAR |  |
| 3 | 5 | C2,C5,C7, C10,C12 | 10u 50V | CAP_EL 80 65 31 RAD 200 | Capacitor 3.3V, 24V (10uF, 50V, 105C, 1000hr) | P5567-ND | Digikey | Panasonic: ECA-1HHG100 |  |
| 4 | 2 | D2,D3 | 5KP7.5A | DIODE_6A 600 200 63 AX 21 | DIODE TVS 7.5V 5000W AXL UNI 5% | 5KP7.5ALFCT-ND | Digikey | LittleFuse: 5KP7.5A | Vishay: 5KP7.5A |
| 5 | 2 | FS2,FS4 | 1.0A |  | Fuse (24V, 1.0A, 250V TR5) | WK3048BK-ND | Digikey | Littelfuse. Wickmann: 37311000410 |  |
| 6 | 2 | FS3,FS5 | 2.0A |  | Fuse (3.3V, 2.0A, 250V TR5) | WK3057BK-ND | Digikey | Littelfuse. Wickmann: 37312000410 |  |
| 7 | 1 | FS1 | 6.3A |  | Fuse (24V, 6.3A, 250V TR5) | WK3066BK-ND | Digikey | Littelfuse. Wickmann: 37316300410 |  |
| 8 | 5 | F1,F2,F3, F4,F5 | 56000001319 | TR5 | Fuseholder TR5 machine contacts | WK0001-ND | Digikey | Littelfuse. Wickmann: 56000001319 |  |
| 9 | 1 | J1 | 0015246243 | 15-24-6243 SPECIAL | Header, 24 POS, 4.2mm, 600VAC/600VDC, 5.0A, Male, Rt. Ang, Sel. Gold, Thru-Hole | 15246243 | Arrow | Molex: 0015246243 | Molex: 0015249245 Datasheet: page 147 |
| 10 | 1 | J2 | 1-770966-1 | 1-770966-1 | Header MINI MATE-N-LOK (2 pin, 600VAC/600VDC, 9.5-3A, rt ang, gold, thru-hole) | A32472-ND | Digikey | Tyco: 1-770966-1 Datasheet: page 140 |  |
| 11 | 2 | J3,J4 | 1-770969-1 | 1-770969-1 SPECIAL | Header MINI MATE-N-LOK (6 pin, 600VAC/600VDC, 7-2.5A, rt ang, gold, thru-hole) | A32476-ND | Digikey | Tyco: 1-770969-1 Datasheet: page 140 |  |
| 12 | 6 | Q1,Q2,Q3, Q4,Q5,Q6 | KSP44 | TO92 100 60 31 123 | TRANS BJT NPN SMALL SIGNAL, VCE 400VDC MAX | 512-KSP44TF | Mouser | Fairchild: KSP44TF | On Semi: MPSA44 |

Table 11
Components of DCM Card

| 13 | 3 | R1,R5,R10 | 4.3k 350V 0.5W | RES_1/2W 600 150 36 AX | RES 4.3K OHM 1/2W 5% CARBON FILM | 4.3KH-ND | Digikey | Yageo: CFR-50JB-4K3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 2 | R6,R11 | 300 | RES_1/4W 500 80 31 AX | RES 300 OHM 1/4W 5% CARBON FILM | 300QBK-ND | Digikey | Yageo: CFR-25JB-300R |  |
| 15 | 2 | R4,R9 | 3.9k 750V 3W | RES_3W 1000 100 46 AX | RES 3.9k OHM METAL FILM 3W 750VDC 5% | PPC3.9KW-3JCT-ND | Digikey | Vishay: PR03000203901JAC00 |  |
| 16 | 4 | R2,R3,R7,R8 | 1.6M 3.5kV 0.5W | RES_1/2W 600 150 36 AX | RES 1.6M OHM METAL FILM .50W 3.5kVDC 5% | PPCHJ1.6MCT-ND | Digikey | Vishay: HVR3700001604JR500 |  |
| 17 | 1 | D1 | HLMP-4719 | LED_SSF-LXH100ID | LED, 5MM, YLW DIFFUSED | 516-1326-ND | Digikey | Avago: HLMP-4719 |  |
| 18 | 1 | SU1 | HLMP-5029 |  | HOUSING RA FOR 5MM HIGH DOME LED | 516-1395-ND | Digikey | Avago: HLMP-5029 |  |
| 19 | 2 | LED1,LED2 | 571-0132F | LED_571-01XX | LED IND 2MM DUAL RA YLW/GRN DIFF | 350-1867-ND | Digikey | Dialight: 571-0132F |  |
| 20 | 2 | M1,M2 | 7774 | KEYSTONE-7774 | PCB SCREW TERM MOUNTING BRACKET, 4-40 THREAD | 534-7774 | Mouser | Keystone: 7774 |  |

Table 11 (continued)
Components of DCM Card

## FEB Card

| Item | Qty | Reference | Part | PCB Footprint | Description | Dist Order Code | Distrib. | Primary Manufacturer+ P/N | Secondary Manufacturer+ P/N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4 | C1,C6,C11, C16 | 0.1u 630V | BOXCAP 590 40 710X360 | CAP METAL POLYPRO .1UF 630V 5% | P12158-ND | Digikey | Panasonic: ECW-F6104JL |  |
| 2 | 8 | C2,C4,C7, C9,C12,C14, C17,C19 | 0.1u 50V | DISCCAP 200 80 31 200X100 | Capacitor 3.3V, 24V (0.1uF, 50V) | 478-3193-ND | Digikey | AVX: SR215E104MAR |  |
| 3 | 8 | C3,C5,C8, C10,C13, C15,C18,C20 | 10u 50V | CAP_EL 80 65 31 RAD 200 | Capacitor 3.3V, 24V (10uF, 50V, 105C, 1000hr) | P5567-ND | Digikey | Panasonic: ECA-1HHG100 |  |
| 4 | 4 | D3,D4,D5,D6 | 5KP7.5A | DIODE_6A 600 200 63 AX 21 | DIODE TVS 7.5V 5000W AXL UNI 5% | 5KP7.5ALFCT-ND | Digikey | LittleFuse: 5KP7.5A | Vishay: 5KP7.5A |
| 5 | 4 | FS1,FS3, FS5,FS7 | 1.0A |  | Fuse (24V, 1.0A, 250V TR5) | WK3048BK-ND | Digikey | Littelfuse. Wickmann: 37311000410 |  |
| 6 | 4 | FS2,FS4, FS6,FS8 | 2.0A |  | Fuse (3.3V, 2.0A, 250V TR5) | WK3057BK-ND | Digikey | Littelfuse. Wickmann: 37312000410 |  |
| 7 | 8 | F1,F2,F3, F4,F5,F6, F7,F8 | 56000001319 | TR5 | Fuseholder TR5 machine contacts | WK0001-ND | Digikey | Littelfuse. Wickmann: 56000001319 |  |
| 8 | 1 | J1 | 15246243 | 15-24-6243 SPECIAL | Header, 24 POS, 4.2mm, 600VAC/600VDC, 5.0A, Male, Rt. Ang, Sel. Gold, Thru-Hole | 15246243 | Arrow | Molex: 0015246243 | Molex: 0015249245 Datasheet: page 147 |
| 9 | 4 | J2,J3,J4,J5 | 1-770969-1 | 1-770969-1 SPECIAL | Header MINI MATE-N-LOK (6 pin, 600VAC/600VDC, 7-2.5A, rt ang, gold, thru-hole) | A32476-ND | Digikey | Tyco: 1-770969-1 Datasheet: page 140 |  |
| 10 | 12 | Q1,Q4,Q5, Q6,Q9,Q10, Q11,Q14, Q15,Q16, Q19,Q20 | KSP44 | TO92 100 60 31 123 | TRANS BJT NPN SMALL SIGNAL, VCE 400VDC MAX | 512-KSP44TF | Mouser | Fairchild: KSP44TF | On Semi: MPSA44 |
| 11 | 4 | R7,R14,R21, R28 | 4.3k 350V 0.5W | RES_1/2W 600 150 31 AX | RES 4.3K OHM 1/2W 5% CARBON FILM | 4.3KH-ND | Digikey | Yageo: CFR-50JB-4K3 |  |
| 12 | 4 | R8,R15,R22, R29 | 300 | RES_1/4W 500 80 31 AX | RES 300 OHM 1/4W 5% CARBON FILM | 300QBK-ND | Digikey | Yageo: CFR-25JB-300R |  |
| 13 | 4 | R5,R12,R19, R26 | 3.9k 750V 3W | RES_3W 1000 100 46 AX | RES 3.9k OHM METAL FILM 3W 750VDC 5% | PPC3.9KW-3JCT-ND | Digikey | Vishay: PR03000203901JAC00 |  |
| 14 | 8 | R2,R4,R9, R11,R16, R18,R23,R25 | 1.6M 3.5kV 0.5W | RES_1/2W 600 150 36 AX | RES 1.6M OHM METAL FILM .50W 3.5kVDC 5% | PPCHJ1.6MCT-ND | Digikey | Vishay: HVR3700001604JR500 |  |
| 15 | 4 | D7,D8,D9, D10 | 571-0132F | LED_571-01XX | LED IND 2MM DUAL RA YLW/GRN DIFF | 350-1867-ND | Digikey | Dialight: 571-0132F |  |
| 16 | 2 | M1,M2 | 7774 | KEYSTONE-7774 | PCB SCREW TERM MOUNTING BRACKET, 4-40 THREAD | 534-7774 | Mouser | Keystone: 7774 |  |

Table 12
Components of FEB Card

## High Voltage Patch Panel

| ID | Description | Quantity | Manufacturer / Item # | Distributor / Item # | Comments |
| --- | --- | --- | --- | --- | --- |
| [H1] | Enclosure | 1 | Enclosures And Cases | --- | -rack mount enclosure for a 19" relay rack -height: 1U -material: natural aluminum -dimensional drawings of front and rear panel: see pages 113 - 114 |
| [H2] | PS to patch panel cable | 1 | Kerpen 76130005 (SL-V2YCEH 37XAWG26/7) | ISEG | -37 conductors, stranded, tinned copper -size of each conductor: 26 AWG -wire insulation diameter of each conductor: 0.051" -voltage: 6kV -terminated by manufacturer on one side with a REDEL 51 pin connector (see data sheet on page 156) -single conductors terminated with sockets [H5] in plug [H3] -datasheet: page 201 |
| [H3] / [H3a] | REDEL connector and pins | 1 | --- | --- | - Socket/housing for 51 conductor cable connection. - pins included - available from power supply manufacturer/distributer - datasheet: page 143 |
| [H7a] / [H7b] | HV Wires | 16 x 8" | --- | McMaster-Carr 7587K622 / 7587K161 | -26 AWG -color: orange (for 450V source) / blue (for 450V return) -both wires are used as twisted pairs -single conductor, stranded, tinned copper -voltage rating: 600 VAC -outside diameter: 0.095" -on one side is terminated with pins [H6] in receptacle [H4] -other side is soldered to triaxial bulkheads [H10] (orange at center conductor / blue at intermediate shield) -datasheet: see page 185 |
| [H8] | Safety loop wires | 1 x 2" | --- | McMaster-Carr 7587K589 | -22 AWG -color: Black -single conductor, stranded, tinned copper -voltage rating: 600 VAC -outside diameter: 0.095" -terminated with pins [H6], and used at receptacle [H4] -datasheet: see page 185 |
| [H10] | Triaxial bulkheads | 16 (32) 9 | Pomona 5219 | Allied Electronics 885-6069 | -operating voltage: 500 VRMS -pushed through cutouts of backpanel of enclosure [H1], and mounted in cutouts of insulating sheet [H9] -center conductor (450V source) connected to orange wire [H7a] -intermediate shield (450V return) connected to blue wire [H7b] -ground lug (outer shield) connected to PCB [H19] -datasheet: see page 116 |
| [H11] | Triaxial connector | 32 (64) 9 | Pomona 5218 | Allied Electronics 885-5218 | -operating voltage: 500 VRMS -used to terminate triaxial cables [H12] -datasheet: see page 117 |
| [H13] | Resistors | 16 | Stackpole Electronics HVCB1206JDD1K00 | Digikey HVCB1206JDD1K00CT-ND | -1 k -0.25 W -datasheet: see page 180 |
| [H19] | PCB | 1 | --- | --- | - Designed to provide a safety ground without creating a ground loop between the PDBs - Each connection to ground is isolated by a 1 kΩ resistor |
| [H17a] | Screws | 18 | --- | McMaster-Carr 91772A151 | -pan head, stainless steel -used to mount the PCB [H19] to the inside of the backpanel of enclosure [H1] -#6-32, length: 3/4" |
| [H17b] | Nuts | 18 | --- | McMaster-Carr 91841A007, | -hex, stainless steel -used together with screw [H17a] -18-8, #6-32, 5/16” width |
| [H17c] | Washers | 36 | --- | McMaster-Carr 96659A102 | -unthreded, stainless steel -used together with screw [H17a] -18-8, #6, 3/8” |
| [H17d] | Star washers | 18 | --- | McMaster-Carr 92146A540 | -stainless steel -used together with screw [H17a] -#6, OD 0.32" |

Table 13 Components of the High Voltage Patch Panel

# Low Voltage Breakout Box

| [L1] | 100 Amp Marine Breakers | 3 | Blue Sea Systems 7549 | --- | - Breaker for the 3.3V source line |
| --- | --- | --- | --- | --- | --- |
| [L2] | 60 Amp Marine Breakers | 3 | Blue Sea 7547 | --- | - Breaker for the 24V source line |
| [L3] | 2 AWG Red LV cable | --- | Methode HV2-140-2 | --- | - Internal cable used for the 3.3V source line |
| [L4] | 2 AWG Black LV cable | --- | Methode HV2-140-0 | --- | - Internal cable used for the 3.3V return line |
| [L5] | 6 AWG Yellow LV Cable | --- | Methode HV6-85-4 | --- | - Internal cable used for the 24V source line |
| [L6] | 6 AWG Black LV cable | --- | Methode HV6-85-0 | --- | - Internal cable used for the 24V return line |
| [L7] | 4 conductor cable, 22AWG, gray | --- | Belden 8723 | --- | - Internal cable used for the sense connection |
| [L8] | 6 AWG Marine Connector | 3 | Anderson Power Products 6800G3 | --- | - Connects to the 24V power cables |
| [L9] | 2 AWG Marine Connector | 3 | Anderson Power Product 6326G5 | --- | - Connects to the 3.3V power cables |
| [L10] | SOCKET CONTACT, 20 DF for Sense Cables | 3 | Tyco 66504-9 | Newark 25C1826 | - Pins for the sense connectors |
| [L11] | HDP-20 Receptacle, 9 Position | 3 | Tyco: 205203-3 | Newark 44F8700 | - Sense cable receptacle |
| [L12] | Female Screwlock Assembly Kit | 3 | Tyco: 5205817-3 | Newark 60K8282 | - Hardware for sense cable receptacle |
| [L13] | Ring terminals, 2AWG, ¼” | 12 | Waytek 35020 | --- | - Ring terminals for 3.3V lines |
| [L14] | Ring terminals, 6AWG, ¼” | 12 | Waytek 34001 | --- | - Ring terminals for 24V lines |

Table 14 Componenets of the Low Voltage Breaker Box

# Pictures

## Power Distribution Box

Figure 13
Power Distribution Box Low Voltage Connectors and Backplane with Bus Bars

E3

C1

O2

O3

O20a

(left hole)

C2

C5

C4

C3

C6

O20c

(left hole)

C15a

C17a

E1

(left panel)

Figure 14 Power Distribution Box v. 3.3

C10a/C11

twisted pair

C1

C2

E17

E16

O7

C3

C4

O11

C12

C13

O27

C16a

C12

O28

C14a

C10b/C11

twisted pair

C10c/C11

twisted pair

Figure 15
Power Distribution Box (Low Voltages Breakers and Backplane Connectors)

O6

O7

O5a

C10a/C11

twisted pair

C10b/C11

twisted pair

C10c/C11

twisted pair

O11

C2

C1

O5a

O5b

1

2

3

3 pin housing Tyco 172166-1 [O28]

with sockets Tyco 1-770988-0 [O29]

2 pin housing Tyco 172165-1 [O27] with sockets Tyco 1-770988-0 [O29]

1

2

Figure 16
Power Distribution Box (Ground Connectos, and Backplane with Bus Bars)

E3

O18

C9

C8

O20b

O19b

O20c

O19b

(right hole)

C3

C4

C7

O20a

O19a (right hole)

E1 (right panel)

O20b (right hole)

Triaxial Connector

Pomona 5219 [O1]

intermediate shield

center contact

Figure 17
Power Distribution Box (Low Voltage Connectors and Back Panel with High Voltage Connector and Sense Cable Connector)

C14b

C15b

C17b

C16b

O4a

1

9

6

2

3

4

5

7

8

6 pin housing Tyco 205203-3 [O4a] with sockets Tyco 66504-9 [O4b]

O8

O10

E9

E3

C17a

C16b

C16a

C15b

C15a

C14b

C14a

C10a/C11

twisted pair

O1

O4c

O10

O1

O4a

O4c

O2

O3

O18

Figure 18
Power Distribution Box (Backpanel)

E3

Backplane

Figure 19 Backplane

Figure 20 Backplane
(first 5 slots from the left side)

24V connector

HV connector

Connector for indicator card

Connector for DCM card

Connector for 1st FEB card

Connector for 2nd FEB card

Connector for 3rd FEB card

## Indicator Card

Figure 21 Indicator Card

Figure 22 Indicator Card

## DCM Card

Figure 23 DCM Card

6 pin connectors for FEB cables

TEC (24V) Indicators

FEB (3.3V) Indicators

2 pin connector for DCM cable

DCM (24V) Indicator

Figure 24 DCM Card Front Panel

## FEB Card

6 pin connectors

for FEB cables

TEC (24V) Indicators

FEB (3.3V) Indicators

Figure 25 FEB Card

Figure 26 FEB Card

## High Voltage Patch Panel

Figure 27
High Voltage Power Supply and High Voltage Patch panel.

center contact (HV Source)

intermediate shield (HV Return)

ground lug (outer shield)

H10

H10

H19

H1 (back panel)

H1 (front panel)

Figure 28
Picture of HV patch panel with top cover removed

Triaxial Connector Pomona 5219 [H10]

H8

H7a/H7b

36 positions receptacle Tyco 207020-1 [H4] with pins Tyco 66566-4 [H6]

H3

H4

H14

H13

Figure 29 A closer view of the Patch Panel PCB. The silk screen indicates the HV channel and the pinout to the REDEL connector.

Figure 30 An unpopulated Patch Panel PCB

Figure 31 The rear panel of the High Voltage Patch Panel, with channel and PDB position labels.

H8 (hidden)

H6

H7a/H7b

H18a

H10

H17a

H16

H1 (back panel)

H1 (front panel)

Channel 1

Channel 8

Channel 7

Channel 15

## Low Voltage Breakout Box

chassis gets mounted on the mounting rails of the relay rack

Figure 32 Chassis mounted in a relay rack

chassis

LV power supply

Figure 33 PL506 power supply and chassis

# Figure 34 Chassis with power supply installed in relay rack.

support angle brackets do not block the airflow of the power supply

LV power supply

chassis

breaker box

# Figure 35 Underside of chassis mounted in relay rack.

sense cable connector for channels 0 and 1

return terminal for channel 0 (3.5V)

back of chassis

source terminal for channel 0 (3.5V)

source and return terminals for channel 1 (24V)

100A breaker for channel 0

60A breaker for channel 1

Marine connectors

sense cable connectors for channels 0 and 1

Figure 36 Inside of the breaker box.

Marine connector for channel 0 (3.5V)

Marine connector for channel 1 (24V)

sense cable connector for channels 1 and 2

breaker box panel

cover of the breaker box

100A breaker for channel 0 (3.5V)

60A breaker for channel 1 (24V)

# Figure 34 Back of the breaker box

inside of the breaker box

back of the breaker box

LV power supply pushed into the chassis in the relay rack

bottom of chassis mounted in relay rack

chassis mounted in relay rack

power supply and chassis

# Figures and Schematics

## Power Distribution Box

Figure 38
Circuit Diagram for Power Distribution System (incl. Power Distribution Box)

## Backplane

Figure 39 Circuit Diagram for Backplane

Figure 39 (continued) Circuit Diagram for Backplane

## Indicator Card

Figure 40 Circuit Diagram for Indicator Card

## DCM Card

Figure 41 Circuit Diagram for DCM Card

Figure 41 (continued) Circuit Diagram for DCM Card

Figure 41 (continued) Circuit Diagram for DCM Card

## FEB Card

Figure 42 Circuit Diagram for FEB Card

Figure 42 (continued) Circuit Diagram for FEB Card

Figure 42 (continued) Circuit Diagram for FEB Card

Figure 42 (continued) Circuit Diagram for FEB Card

Figure 42 (continued) Circuit Diagram for FEB Card

## High Voltage Patch panel

mates with socket housing Tyco 207019-1 of the 37 conductor cable which goes to the HV PS

G10/FR4 insulation sheet

pin housing Tyco 207020-1

aluminum chassis

triaxial bulkhead Pomona 5219

mates with triaxial connector Tyco 207019-1 of the triaxial cable which goes to the PDB

inner conductor (source)

intermediate shield (return)

outer shield

ground connector stud

twisted pair

safety loop

ground wire

1 k

1 k

1 k

1 k

1 k

1 k

no contact between outher shield and chassis

Figure 43
Circuit diagram for HV patch panel.

The dashed lines indicate the parts which are not present if only one powers supply is used (and therefore only 16 channels)

Figure 44 Schematic of the Patch Panel PCB

# Datasheets, Dimensional Drawings, etc.

## Power Supplies

### Datasheet for LV power supply Wiener PL506

Datasheet for LV power supply Wiener PL506 (continued)

Datasheet for LV power supply Wiener PL506 (continued)

Datasheet for LV power supply Wiener PL506 (continued)

Datasheet for LV power supply Wiener PL506 (continued)

### Drawing of connectors on back of LV power supply Wiener PL506

### Datasheet for HV power supply ISEG EHS F6-05x 156-F

Datasheet for HV power supply ISEG EHS F6-05x 156-F (continued)

Datasheet for HV power supply ISEG EHS F6-05x 156-F (continued)

Datasheet for HV power supply ISEG EHS F6-05x 156-F (continued)

## Power Distribution Box Enclosure

### Schroff Subrack 19”, 3U

### Dimensional Drawing for left side panel Schroff 34560-193 and modification

### Dimensional Drawing for right side panel Schroff 34560-193 
and modification

### Dimensional Drawing for back panel Schroff 20848-097 and modifications

### Dimensional Drawing for Switch Panel

### Dimensional Drawing for Ground Connector Bar

## High Voltage Patch panel Enclosure

### Dimensional drawing of Front Panel of High Voltage Patch panel

### Dimensional drawing of back panel

## Connectors

### Datasheet for triaxial bulkhead Pomona 5219

### Datasheet for triaxial connector Pomona 5218

Datasheet for triaxial connector Pomona 5218 (continued)

### Datasheet for Low Voltage Connector APP 6800G3

Datasheet for Low Voltage Connector APP 6800G3 (continued)

### Datasheet for Low Voltage Connector APP 6326G5

Datasheet for Low Voltage Connector APP 6326G5 (continued)

### Datasheet for Sense Cable Connector 
Tyco 205203-3 (receptacle – socket housing) and 
Tyco 205204-4 (plug – pin housing)

Datasheet for Sense Cable Connector
Tyco 205203-3 (receptacle – socket housing) and 
Tyco 205204-4 (plug – pin housing) (continued)

Datasheet for Sense Cable Connector
Tyco 205203-3 (receptacle – socket housing) and 
Tyco 205204-4 (plug – pin housing) (continued)

### Datasheet for Sockets Tyco 66504-9 and Pins Tyco 66506-9

Datasheet for Sockets Tyco 66504-9 and Pins Tyco 66506-9 (continued)

### Datasheet for Screwblock Assemby Kit Tyco 5205817-3

Datasheet for Screwblock Assemby Kit Tyco 5205817-3 (continued)

### Datasheet for Cable Clamps Tyco 5748676-1 and Jackscrew Tyco 747784-8

Datasheet for Cable Clamps Tyco 5748676-1 and 
Jackscrew Tyco 747784-8 (continued)

### Datasheet for DCM Cable Plug Tyco 172165-1, 
HV Cable Plug Tyco 172166-1, and FEB Cable Plug Tyco 172168-1

Datasheet for DCM Cable Plug Tyco 172165-1, 
HV Cable Plug Tyco 172166-1, and FEB Cable Plug Tyco 172168-1 (continued)

Datasheet for DCM Cable Plug Tyco 172165-1, 
HV Cable Plug Tyco 172166-1, and FEB Cable Plug Tyco 172168-1 (continued)

### Datasheet for Connector Sockets Tyco 1-770988-0

Datasheet for Connector Sockets Tyco 1-770988-0 (continued)

### DCM and HV Backplane Connectors Tyco 1-770872-1 and 1-770873-1

DCM and HV Backplane Connectors Tyco 1-770872-1 and 1-770873-1 (continued)

DCM and HV Backplane Connectors Tyco 1-770872-1 and 1-770873-1

### DCM and HV Card Connectors Tyco 1-770966-1 and 1-770969-1

DCM and HV Card Connectors Tyco 1-770966-1 and 1-770969-1 (continued)

DCM and HV Card Connectors Tyco 1-770966-1 and 1-770969-1 (continued)

### Backplane Connectors for Cards Molex 0015247243

Backplane Connectors for Cards Molex 0015247243 (continued)

Backplane Connectors for Cards Molex 0015247243 (continued)

Backplane Connectors for Cards Molex 0015247243 (continued)

### Card Connector for Backplanes Molex 0015249245

Card Connector for Backplanes Molex 0015249245 (continued)

Card Connector for Backplanes Molex 0015249245 (continued)

Card Connector for Backplanes Molex 0015249245 (continued)

### Datasheet for Crimp-On Wire Connectors 
McMaster-Carr 7242K111 and 7242K64

### Datasheet for Ring Terminals Waytek 34001, 35020, and 35022

### Datasheet for Ring Terminal Tyco 330301

### Datasheet for Ring Terminals Allied 400-0002

### Datasheet for ring terminal Panduit PV18-14R-CY

### Datasheet for connector REDEL SAG.H51

Datasheet for connector REDEL SAG.H51 (continued)

Datasheet for connector REDEL SAG.H51 (continued)

Datasheet for connector REDEL SAG.H51 (continued)

### Datasheet for sockets/pins Tyco 66109-4/66566-4

Datasheet for sockets/pins Tyco 66109-4/66566-4

## Switches

### Datasheet for HV Switch Sprecher+Schuh D7M-SM22, D7-X10, D7-ALM
(McMaster-Carr 9209K551, 9209K127)

Datasheet for HV Switch Sprecher+Schuh D7M-SM22, D7-X10, D7-ALM 
(continued)

Datasheet for HV Switch Sprecher+Schuh D7M-SM22, D7-X10, D7-ALM
(continued)

Datasheet for HV Switch Sprecher+Schuh D7M-SM22, D7-X10, D7-ALM
(continued)

Datasheet for HV Switch Sprecher+Schuh D7M-SM22, D7-X10, D7-ALM
(continued)

### Datasheet for low voltage breakers BS 7547, BS 7549

Datasheet for low voltage breakers BS 7547, BS 7549 (continued)

## Fuses, Resistors, etc.

### Datasheet for Fuseholder LittelFuse 03453LS2HXLN

### Datasheet for Fuse Holder LittelFuse 03453LS2HXLN (continued)

Datasheet for Fuse LittelFuse 0314.500HXP

Datasheet for Fuse LittelFuse 0314.500HXP (continued)

Datasheet for Fuse LittelFuse 0314.500HXP (continued)

### Datasheet for 1k resistor HVCB1206JDD1K00CT-ND

## Cables

### Datasheet for 2AWG and 6AWG low voltage cables CableCo 
HV6-85-4, HV6-85-0, HV6-140-2, HV6-140-0

### Datasheets for HV cables
 McMaster-Carr

### Datasheets for DCM cables 
McMaster-Carr 7587K932, 7587K931, 7587K45, and 7587K934

### Datasheets for Sense cables McMaster-Carr 7587K932, 7587K931, 7587K45, and 7587K934

### Datasheets for 2AWG and 6AWG cables from Tristar

### Datasheet for triaxial cable Belden 9222 00450

Datasheet for triaxial cable Belden 9222 00450 (continued)

Datasheet for triaxial cable Belden 9222 00450 (continued)

### Datasheet for Sense Cable Belden 8723

Datasheet for Sense Cable Belden 8723 (continued)

Datasheet for Sense Cable Belden 8723 (continued)

### Datasheet for 6 conductor cable Belden 27600A

Datasheet for 6 conductor cable Belden 27600A (continued)

Datasheet for 6 conductor cable Belden 27600A (continued)

### Datasheet for DCM cable ADC 911802

### Datasheets for Ground Braids Erico 557240 and Lugs Erico 557180

### Datasheet for the HV cable Kerpen 76130005

Datasheet for the HV cable Kerpen 76130005 (continued)

## Dimensional Drawings for Busbars, Backplane and Cards

### Dimensional Drawing for Source Bus Bar

### Dimensional Drawing for Return/Ground Bus Bar

### Dimensional Drawing for Backplane

### Dimensional Drawing for Indicator Card

### Dimensional Drawing for DCM Card

### Dimensional Drawing for FEB Card

## Faceplates for Cards

### Dimensional Drawing for indicator front panel

### Dimensional Drawing for DCM front panel

### Dimensional Drawing for FEB front panel

Electrical Design Standards for Electronics to be used in Experiment Apparatus at Fermilab, April 15, 1999 (unpublished).

http://www-esh.fnal.gov/pls/default/esh_home_page.page?this_page=5040

There are also two spare FEB connectors at the DCM card.

Electrical Design Standards for Electronics to be used in Experiment Apparatus at Fermilab, April 15, 1999 (unpublished).

 Power Distribution Box High Voltage Current Limiter Evaluation, Hunter Tammaro, E. Craig Dukes, and Larry St. John, 14 August, 2009, NOA internal document, DocDB-4154-v1.

Fault Analysis: Single Point Failures for the 3-Transistor Current Limiter, John Oliver, 15 October, 2009, NOA internal document, DocDB-4526-v1.

See also cable section

Cables [C10a],[C10b],[C10c] form a twisted pair with cable [C11].

This is not the current wire specification.